# 2023 Alfa Romeo Giulia

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Vehicle Type	Mid-size sedan, RWD/AWD

Assembly Plant	Cassino Assembly Plant, Cassino, Italy

EPA Vehicle Class	Mid-size car

### BODY/CHASSIS

Layout	Longitudinal front engine, RWD/AWD

Construction	Unitized high-strength steel body

Aluminum front and rear vehicle frames, front shock towers, brakes, suspension components, doors and fenders

Composite rear cross member

Carbon fiber driveshaft

Aluminum roof and hood (Sprint, Ti, Veloce)

Carbon fiber roof and hood (Quadrifoglio exclusive)

### ENGINE: ALL-ALUMINUM 2.0-LITER TURBOCHARGED 4-CYLINDER 280HP

Availability	Standard on Giulia Sprint, Giulia Ti, Giulia Veloce

Type and Description	Inline-four, direct injection, turbo

Displacement	121.7 cu. in. (1,993 cc)

Bore x Stroke	3.31 x 3.54 (84.0 x 89.9)

Valve System	MultiAir2, SOHC, four valves per cylinder with 
silent chain driven timing drive

Hollow assembled exhaust camshaft with end pivot roller finger followers and hydraulic lash adjusters

Turbocharger	Direct mount twin-scroll turbocharger with electric wastegate actuation. Intake manifold with liquid cooled charge air cooler.

Fuel Injection	Gasoline direct injection with injector centrally located 
in the combustion chamber

Max injection pressure 2,900 psi

### ENGINE: ALL-ALUMINUM 2.0-LITER TURBOCHARGED 4-CYLINDER 280HP (continued)

Construction	Cylinder head: permanent mold air quenched cast aluminum alloy with integrated exhaust manifold

Crankcase: precision and cast aluminum alloy with casted steel liners

Crankshaft: super-finished forged nitride steel with journals offset from cylinder bore centerline

Piston: forged aluminum cooled with pressure actuated oil jets

Power (SAE net)	280 hp (209 kW) @ 5,200 rpm (140 hp/liter)

Torque (SAE net)	306 lb.-ft. (415 Nm) @ 2,000 - 4,800 rpm

Redline on the Instrument Cluster	5,500 rpm

Max. Engine Speed	6,200 rpm

Fuel Requirement	Premium unleaded, 91 octane (R+M)/2

EPA Fuel Economy mpg (city/hwy/combined)	24/33/27 (RWD)

23/31/26 (AWD)

Engine Assembly Plant	Termoli, Italy

### ENGINE: ALL-ALUMINUM 2.9-LITER TWIN-TURBOCHARGED V-6 505HP RWD

Availability	Giulia Quadrifoglio

Type and Description	V-6, 90-degree, direct injection

Displacement	176.4 cu. in. (2,891 cm3)

Bore x Stroke	3.40 x 3.23 (86.5 x 82.0)

Valve System	Chain-driven DOHC with intake and exhaust continuous 
camshaft phasing

Four valves per cylinder roller finger actuated with hydraulic 
lash adjusters

Mechanical cylinder deactivation on right bank

Turbochargers	Integrated IHI single-scroll turbo manifold with water charge air cooler

Fuel Injection	Port fuel injection and side mounted gasoline direct injection

Max injection pressure 3,600 psi

Construction	Cylinder head and cover: aluminum alloy with integrated cam carrier

Crankcase: closed deck aluminum alloy with wet steel liner

Bedplate: aluminum alloy with integrated blow-by control reed valves

Crankshaft: super-finished forged nitride steel with single con-rod pin

Piston: forged aluminum cooled with twin oil jets

Compression Ratio	9.3:1

Power (SAE net)	505 hp (377 kW) @ 6,500 rpm (175 hp/liter)

Torque (SAE net)	443 lb.-ft. (600 N•m) @ 2500-5500 rpm

### ENGINE: ALL-ALUMINUM 2.9-LITER TWIN-TURBOCHARGED V-6 505HP RWD (CONTINUED)

Redline on the Instrument Cluster	6,500 rpm

Max. Engine Speed	7,400 rpm

Fuel Requirement	Premium unleaded, 91 octane (R+M)/2

EPA Fuel Economy mpg (city/hwy/combined)	17/25/20

Engine Assembly Plant	Termoli, Italy

### TRANSMISSION: 8HP50 EIGHT-SPEED AUTOMATIC

Availability	Giulia Sprint, Giulia Ti and Giulia Veloce

Gearbox type	AT8

Gearbox family	8HP50

Gear Ratios

1st	5.00

2nd	3.20

3rd	2.14

4th	1.72

5th	1.31

6th	1.00

7th	0.82

8th	0.64

Transmission Type	RWD/AWD

Final Drive Ratio	3.15

### TRANSMISSION: 8HP75 EIGHT-SPEED AUTOMATIC

Availability	Giulia Quadrifoglio

Gearbox type	AT8

Gearbox family	8HP75

Gear Ratios

1st	5.00

2nd	3.20

3rd	2.14

4th	1.72

5th	1.31

6th	1.00

7th	0.82

8th	0.64

### TRANSMISSION: 8HP75 EIGHT-SPEED AUTOMATIC (CONTINUED)

Transmission Type	RWD

Final Drive Ratio	3.09

### SUSPENSION

Front	Double wishbone with semi-virtual steering axle

Rear	Alfa Link design with vertical rod link (patented design)

Active Damping	Standard on Giulia Quadrifoglio

Included with Performance packages on Giulia Veloce

### STEERING

Type	Rack and pinion with Variable Electric Assist

Ratio	11.8:1

### BRAKES

Standard brake system	Standard on Giulia Sprint, Giulia Ti and Giulia Veloce

Front

Rotor size and type	13.0 x 1.1 (330 x 28)

Caliper size and type	Brembo four-piston calipers

Rear

Rotor size and type	12.5 x 0.86 (320 x 22)

Caliper size and type	Single-piston calipers

High-performance Brembo brake system	Standard on Giulia Quadrifoglio

Front

Rotor size and type	14.2 x 1.26 (360 x 32) vented

Caliper size and type	Brembo six-piston aluminum-monoblock calipers

Rear

Rotor size and type	13.8 in x 1.1 (350 x 28) vented

Caliper size and type	Brembo four-piston aluminum-monoblock calipers

Ultra-high-performance Brembo 
Carbon Ceramic Material (CCM) brake system	Optional on Giulia Quadrifoglio

Front

Rotor size and type	15.4 x 1.3 (390 x 36) two-piece, vented carbon ceramic matrix

Caliper size and type	Brembo six-piston aluminum-monoblock calipers

Rear

Rotor size and type	14.2 x 1.1 (360 x 28) two-piece, vented carbon ceramic matrix

Caliper size and type	Brembo four-piston aluminum-monoblock calipers

### DIMENSIONS AND CAPACITIES

Wheelbase	111.0 (2,820)

Track, Front	61.3 (1,557) – Giulia Sprint, Giulia Ti and Giulia Veloce

61.2 (1,555) – Giulia Quadrifoglio

Track, Rear	64.0 (1,625) – Giulia Sprint, Giulia Ti and Giulia Veloce

63.3 (1,607) – Giulia Quadrifoglio

Overall Length	182.8 (4,643) – Giulia Sprint, Giulia Ti and Giulia Veloce

182.6 (4,639) – Giulia Quadrifoglio

Overall Width (not including mirrors)	73.2 (1,860) – Giulia Sprint, Giulia Ti and Giulia Veloce

73.8 (1,874) – Giulia Quadrifoglio

Overall Height	56.5 (1,436) – Giulia Sprint, Giulia Ti and Giulia Veloce (RWD)

57.1 (1,450) – Giulia Sprint, Giulia Ti and Giulia Veloce (AWD)

56.1 (1,426) – Giulia Quadrifoglio

Drag Coefficient (cd)	0.28 – Giulia Sprint, Giulia Ti and Giulia Veloce

0.31 – Giulia Quadrifoglio

Fuel Tank Capacity, gal. (liter)	15.3 (58)

Curb Weight, lbs. (kg)	3,522 (1,598) – Giulia RWD

3,622 (1,643) – Giulia AWD

3,820 (1,733) – Giulia Quadrifoglio RWD

### ACCOMMODATIONS

Seating Capacity, F/R	2/3

Headroom, F/R	38.6/37.6

Legroom, F/R	42.4/35.1

Shoulder Room, F/R	56.1/53.6

SAE Interior Volume (cu. ft.)	95.4 cu. ft.

###