# 2023 Chrysler 300

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Vehicle Type	E-segment sedan, rear-wheel drive (RWD), all-wheel drive (AWD)

Assembly Plant	Brampton Assembly Plant, Ontario, Canada

EPA Vehicle Class	Large car

Introduction Date	2015 as a 2015 model

### BODY/CHASSIS

Layout	Longitudinal front engine, RWD or AWD

Construction	Unitized steel body and aluminum hood

### ENGINE: 3.6-LITER PENTASTAR V-6

Availability		Standard — 300 Touring (RWD/AWD), 300 Touring L (RWD/AWD), 300S (RWD)

Type and Description	60-degree V-type, liquid-cooled

Displacement 	220 cu. in. (3,604 cc)

Bore x Stroke	3.78 x 3.27 (96.0 x 83.0)

Valve System	Chain-driven DOHC, 24 valves, hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block with aluminum-alloy heads

Compression Ratio	10.2:1

Power (SAE net)	292 bhp (218 kW) @ 6,350 rpm

300 bhp (224 kW) @ 6,350 rpm — 300S only

Torque (SAE net)	260 lb.-ft. (353 N•m) @ 4,800 rpm

264 lb.-ft. (358 N•m) @ 4,800 rpm — 300S only

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane (R+M)/2

Oil Capacity	6 quarts (5.7 liters)

Coolant Capacity	14 quarts (13.25 liters)

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors and internal engine features

### ENGINE: 3.6-LITER PENTASTAR V-6 (CONTINUED)

Max. Gross Trailer Weight (unbraked trailer)	1,000 lbs. (454 kg)

EPA Fuel Economy mpg (city/hwy)	TBD (RWD); TBD (AWD)

Engine Assembly Plant	Trenton South Engine Plant, Trenton, Michigan

### ENGINE: 5.7-LITER HEMI® V-8 WITH FUEL SAVER TECHNOLOGY

Availability	Optional — 300S RWD

Type and Description	90-degree V-type, liquid-cooled with variable-cam timing (VCT)

Displacement	345 cu. in. (5,654 cc)

Bore x Stroke	3.92 x 3.58 (99.5 x 90.9)

Valve System	Pushrod-operated overhead valves, 16 valves, eight deactivating 
and eight conventional hydraulic lifters, all with roller followers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.5:1

Power (SAE net)	363 bhp (270 kW) @ 5,200 rpm

Torque (SAE net)	394 lb.-ft. (534 N•m) @ 4,200 rpm

Max. Engine Speed	5,800 rpm (electronically limited)

Fuel Requirement	Unleaded midgrade, 89 octane (R+M)/2 — recommended

Unleaded regular, 87 octane (R+M)/2 — acceptable

Oil Capacity	7 quarts (6.6 liters)

Coolant Capacity	14.7 quarts (13.9 liters)

Emission Controls 	Dual close-coupled three-way catalytic converters, quad heated oxygen sensors and internal engine features

Max. Gross Trailer Weight	1,000 lbs. (454 kg)

EPA Fuel Economy mpg (city/hwy/combined)	TBD

### TRANSMISSION: TORQUEFLITE 8HP50 EIGHT-SPEED AUTOMATIC

Availability	Standard with 3.6-liter Pentastar V-6 engine, RWD

Description	Adaptive electronic control and available paddle-shifted driver-interactive manual control (Sport mode on 300S only). Five-clutch pack design with only two open clutches in any gear. Off-center line pump with low-viscosity oil for reduced spin loss. Torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

### TRANSMISSION: TORQUEFLITE 8HP50 EIGHT-SPEED AUTOMATIC (CONTINUED)

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.30

Final Drive Ratio	2.62

Overall Top Gear	1.76

### TRANSMISSION: TORQUEFLITE 850RE EIGHT-SPEED AUTOMATIC

Availability	Standard with 3.6-liter Pentastar V-6 engine, AWD

Description	Adaptive electronic control and available paddle-shifted driver-interactive manual control (Sport mode on 300S only). Five-clutch pack design with only two open clutches in any gear. Off-center line pump with low-viscosity oil for reduced spin loss. Torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.30

Final Drive Ratio	3.07

Overall Top Gear	2.05

### TRANSMISSION: TORQUEFLITE 8HP70 EIGHT-SPEED AUTOMATIC

Availability	Standard with 5.7-liter engine

Description	Adaptive electronic control, configurable drive modes and paddle-shifted driver-interactive manual control, five-clutch pack design with only two open clutches in any gear, off-center line pump with low-viscosity oil for reduced spin loss, torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.11

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.32

Final Drive Ratio	TBD

Overall Top Gear	1.75

### TRANSFER CASE

Availability	Standard — AWD models

Type	Active, fully variable with front-axle disconnect

Center Differential	Planetary

Torque Split, front/rear	Fully variable

### ELECTRICAL SYSTEM

Alternator	160-amp — standard on 300 Touring and 300 Touring L

180-amp — standard on 300S

Battery	H7 Case, 730 CCA, maintenance-free

### STEERING

Type	Electric power steering (EPS) with multi-mode assist

Overall Ratio	15.5:1 — V-6 (RWD)

16.5:1 — V-6 (AWD)

15.5:1 — V-8 (RWD)

Turn Circle (curb-to-curb)	37.4 ft.

Steering Turns (lock-to-lock)	2.8 — V-6 (RWD)

3.1 — V-6 (AWD)

2.8 — V-8 (RWD)

### SUSPENSION

Front 	Independent SLA with high upper “A” arm

Lateral and diagonal lower links (RWD models), one-piece lower-control arms (AWD models)

Coil spring over gas-charged monotube shock absorbers

32 mm hollow stabilizer bar

Rear	Five-link independent with coil springs

Link-type 19 mm hollow stabilizer bar and isolated suspension cradle

Gas-charged monotube shock absorbers

Isolated suspension cradle

Touring-tuned Suspension 	Standard on 300 Touring, 300 Touring L

Performance-tuned Suspension	Standard on 300S

AWD Touring-tuned Suspension 	Standard on 300 Touring AWD and 300 Touring L AWD

### DIMENSIONS AND CAPACITIES

Wheelbase	120 (3,048)

Track, Front	63.4 (1,611)

Track, Rear	63.8 (1,620)

Overall Length	198.6 (5,044)

Overall Width	75.0 (1,902)

Overall Height	58.5 (1,485) — 300 (RWD with 18-in. tires)

58.7 (1,492) — 300 (RWD with 20-in. tires)

59.2 (1,503) — 300 (AWD with 19-in. tires)

Frontal Area, sq. ft. (sq. m)	26 (660)

26.1 (663) — AWD

Ground Clearance	4.7 (120) — 300 (RWD with 18-in. tires)

5.0 (127) — 300 (RWD with 20-in. tires)

4.8 (122) — 300 (AWD with 19-in. tires)

Drag Coefficient (Cd)	0.320

Curb Weight, lb. (kg)	4,013 (1,820) — V-6 (RWD)

4,267 (1,935) — V-6 (AWD)

4,380 (1,987) — V-8 (RWD)

Weight Distribution, percent front/rear	52/48 — V-6 (RWD)

53/47— V-6 (AWD)

53/47— V-8 (RWD)

Fuel Tank Capacity, gallons (liters)	18.5 (70.0)

### ACCOMMODATIONS

Seating Capacity, front/rear	2/3

Front

Headroom	38.6 (981)

36.9 (936.6) with dual-pane sunroof

Legroom	41.8 (1,061)

Shoulder room	59.5 (1,510)

Hip room	56.2 (1,428)

Seat travel	Driver — 10.6 (270)

Passenger — 8.66 (220)

Recliner angle range, deg.	Power — 70.7

Manual — 70.0

EPA front row interior volume, cu. ft. (cu. m)	55.6 (1.57)

Rear Seat

Headroom	37.9 (963)

36.9 (937) with dual-pane sunroof

Legroom	40.1 (1,019)

Knee clearance	3.9 (99)

Shoulder room	57.7 (1,465)

Hip room	56.1 (1,425)

EPA second-row interior volume, cu. ft. (cu. m)	50.7 (1.44)

Total interior volume, cu. ft. (cu. m)	106.3 (3.01)

EPA Luggage Compartment Volume, cu. ft. (cu. m)	16.3 (0.46)

EPA Interior Volume Index, cu. ft. (cu. m)	122.6 (3.47)

Trunk Liftover Height	30.1 (769)

### TIRES

Availability	Standard on 300 Touring

Size and type	P215/65R17 low-rolling resistance

Mfr. and model	Michelin Energy Saver

Revs per mile (km)	745 (463)

Availability	Standard on 300 Touring AWD and 300 Touring L AWD

Size and type	235/55R19 BSW all-season performance

Mfr. and model	Michelin Primacy MXM4

Revs per mile (km)	715 (444)

### TIRES (CONTINUED)

Availability	Optional on 300 Touring AWD and 300 Touring L AWD

Size and type	235/55R19 BSW all-season performance

Mfr. and model	Pirelli P Zero

Revs per mile (km)	711 (442)

Availability	Standard on 300S; Optional on 300 Touring and 300 Touring L (included with Sport Appearance and Chrome Appearance)

Size and type	245/45R20 BSW all-season performance

Mfr. and model	Firestone Firehawk GTV

Revs per mile (km)	726 (449)

### WHEELS

Availability	Standard on 300 Touring

Type and material	Fine Silver

Size	17 x 7 in.

Availability	Optional on 300 Touring AWD; Standard on 300 Touring L AWD 
(included with AWD and Sport Appearance)

Type and material	Black Noise

Size	19 x 7.5 in.

Availability	Standard on 300 Touring AWD

Type and material	Polished aluminum

Size	19 x 7.5 in.

Availability	Standard on 300S and 300 Touring L; Optional on 300 Touring (included with Sport Appearance)

Type and material	Black Noise

Size	20 x 8.0 in.

### BRAKES

Touring Brake Package 	Standard on 300 Touring, 300 Touring L and 300S

Front

Rotor size and type	12.6 x 1.1 (320 x 28) vented

Caliper size and type	2.36 (60) single-piston sliding with aluminum housing

Swept area	210 sq. in. (1,358 sq. cm)

Rear

Rotor size and type	12.6 x 0.4 (320 x 10) solid

Caliper size and type	1.65 (42) single-piston sliding with aluminum housing

Swept area	264 sq. in. (1,703 sq. cm)

### BRAKES (CONTINUED)

Performance Brake Package 	Standard on 300S

Included with 5.7-liter HEMI® V-8 engines and AWD

Front

Rotor size and type	13.6 x 1.26 (345 x 28) vented

Caliper size and type	1.65 (42) dual-piston sliding with aluminum housing

Swept area	249 sq. in. (1,606 sq. cm)

Rear

Rotor size and type	12.6 x 0.87 (320 x 22) vented

Caliper size and type	1.65 (42) single-piston sliding with aluminum housing

Swept area	264 sq. in. (1,703 sq. cm)

Four-wheel Anti-lock Brake System (ABS) and 
Traction Control	Standard

Electronic Stability Control (ESC) and Brake Assist	Standard

Brake Knockback Mitigation	Standard

Hill-start Assist (HSA)	Standard

Three-mode (Normal, Sport, Off) ESC/Traction 
Control Mode Selection	Standard on 300S

Power-assist Type	8 + 9 (203 + 209) tandem-diaphragm vacuum booster

###