# 2023 Chrysler Pacifica

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### General Information

Vehicle Type	Multipurpose vehicle

Assembly Plant	Windsor, Ontario, Canada

EPA Vehicle Class	Multipurpose vehicle

Introduction Date	2016 as a 2017 model

### BODY AND CHASSIS

Layout	Transverse front engine, front-wheel drive (FWD) or all-wheel drive (AWD)

Construction	Steel unibody with hinged front doors; aluminum-skinned sliding left- and right-side doors — power available; all-aluminum rear liftgate with gas props — power available

### Engine: 3.6-Liter Pentastar V-6

Availability	Standard on gas engine models

Type and Description	60-degree dual overhead cam engine

Displacement	220 cu. in. (3,605 cu. cm)

Bore x Stroke	96 mm x 83 mm

Valve System	24-valve, end pivot roller finger followers with variable-valve lift (intake only) and continuous variable-valve timing on both intake and exhaust cams; chain-driven

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	High-pressure diecast A380 aluminum block with iron liners and 
semi-permanent mold A319 aluminum heads

Compression Ratio	11.3:1

Power (SAE net)	287 hp (214 kW) @ 6,400 rpm

Torque (SAE net)	262 lb.-ft. (355 N•m) @ 4,000 rpm

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane, or E0 to E15

Oil Capacity	5 quarts (4.7 liter)

Coolant Capacity	7.2 quarts (6.8 liter)

### Engine: 3.6-Liter Pentastar V-6 (continued)

Emission Controls	Integrated cast-aluminum header manifolds; positive crankcase ventilation; intake and exhaust cam phasers with internal cooled exhaust gas recirculation; evaporative emissions system

Engine stop-start (ESS) technology — standard

EPA Fuel Economy mpg (city/hwy/combined)	19/28/22 — FWD

17/25/20 — AWD

Assembly Plant	Trenton Engine Plant, Trenton, Michigan

### TRANSMISSION: 948TE TORQUEFLITE NINE-SPEED AUTOMATIC

Availability	Standard on gas engine models

Description	Nine-speed FWD/ AWD, electronically controlled automatic overdrive transmission with torque converter clutch

Clutch-to-clutch architecture with integral electro/hydraulic control module

Gear Ratios

1st 	4.70

2nd	2.84

3rd 	1.91

4th	1.38

5th 	1.00

6th 	0.81

7th 	0.70

8th 	0.58

9th 	0.48

Reverse 	3.81

Spread	9.81

Final Drive Ratio	3.25

AWD	3.73

### Suspension

Front	Independent MacPherson strut, coil over gas-charged shock absorbers, stabilizer bar with hydroformed steel perimeter cradle

Rear	Independent twist-blade with coil springs, twin-tube shock absorbers 
with integrated rebound springs

Rear AWD	Rear stabilizer bar

### STEERING

Type	Electric rack and pinion

Overall Ratio	16.2:1

AWD	15:7:1

Turning Diameter (curb-to-curb)	39.7 ft. (12.1 m)

AWD	40.2 ft. (12.3 m)

Steering Turns (lock-to-lock)	3.13

AWD	3.03

### BRAKES

Power-assist Type	Vacuum assist

Availability	Standard on all models

Front

Size and type 	13.0 x 1.1 (330 x 28) vented rotor with (51.0) twin-piston floating caliper

Swept area 	73.9 sq. in. (476.8 sq. cm)

AWD — size and type	13.8 X 1.1 (350 x 28) vented rotor with (51.0) twin-piston floating caliper

AWD — swept area	81.3 sq. in. (524.8 sq. cm)

Rear

Size and type	13.0 x 0.47 (330 x 12) solid rotor with (44.0) single-piston floating caliper

Swept area	67.6 sq. in. (436.2 sq. cm)

AWD — size and type	13.35 x 0.47 (339 x 12) solid rotor with 44.0 single piston floating caliper

AWD — swept area	71.6 sq. in (461.9 sq. cm)

Parking Brake Type	Electric park brake

Antilock Brake System (ABS)	Standard

Electronic Stability Control (ESC) 	Standard

Traction Control	Standard

Brake Assist	Standard

### Dimensions and Capacities

Wheelbase	121.6 (3,089)

Overhang — Front	38.3 (972)

Overhang — Rear	44.3 (1,127)

Track — Front	68.3 (1,735)

AWD	68.8 (1748)

Track — Rear	68.3 (1,736)

AWD	68.8 (1747)

### Dimensions and Capacities (continued)

Overall Length	204.3 (5189)

Overall Width	79.6 (2,022)

Overall Width With Mirrors	90.4 (2,297)

Overall Height 	69.9 (1,777)

AWD	70.7 (1796)

Liftover Height 	24.3 (617)

AWD	25.1 (637)

Cargo Width at Wheelhouse 	48.8 (1,239)

Maximum Cargo Height 	47.6 (1,208)

Approach Angle (degrees)	14.0

AWD	15.4

Ramp Breakover Angle (degrees)	12.5

AWD	12.8

Departure Angle (degrees)	18.7

AWD	19.8

Ground Clearance 	5.1 (131)

AWD	5.4 (137)

Drag Coefficient (Cd)	N/A

Aero CdA	N/A

Fuel Tank Capacity, gallons (L)	19 (71.9)

Gross Vehicle Weight Rating (GVWR), lbs. (kg)	6,055 (2,747)

AWD	6,300

Towing Capacity, lbs. (kg) 	3,600 (1,633)

EPA Curb Weight, lbs. (kg)		4,521

AWD	4,883

Weight Distribution (percent front/rear)	55/45

### Accommodations

Seating Capacity (front/second/third rows)	2/2/3 or 2/3/3

Total SAE Volume, cu. ft.	197.3

Front

Headroom	40.1 (1,020)

Headroom with sunroof 	38.4 (975)

Legroom	41.1 (1,045)

Shoulder room	63.8 (1,622)

Hip room	59.0 (1,500)

### Accommodations (continued)

Seat travel	8.7 (220)

Recliner angle range (degrees)	58

First row SAE volume, cu. ft. 	61.1

Second Row

Headroom	39.6 (1,006)

Headroom with sunroof 	38.0 (966)

Legroom	39.0 (992)

Minimum knee clearance	4.8 (123)

Shoulder room	63.0 (1,602)

Hip room	64.8 (1,647)

Second row SAE volume, cu. ft.	56.5

Third Row

Headroom	38.7 (984)

Headroom with sunroof 	38.7 (984)

Legroom	36.5 (929)

Knee clearance	3.5 (89)

Shoulder room	61.2 (1,555)

Hip room	49.5 (1,258)

Third row SAE volume, cu. ft.		47.5

Cargo Volume

Maximum passenger volume, cu. ft.	165.0

Maximum SAE volume behind first row, cu. ft.	140.5

Maximum SAE volume behind second row, cu. ft.	87.5

Maximum SAE volume behind third row, cu. ft.	32.3

Total passenger plus volume behind 
third row, cu. ft.	197.3

### Wheels

Availability	Standard on Touring, Touring L – FWD only

Type and material	Cast aluminum

Size	17 x 7

Availability	FWD: standard on Limited, optional on Touring 
(S Appearance Package)

AWD: standard on Touring L and Limited

Type and material	Cast aluminum

Size	18 x 7.5

### Wheels (continued)

Availability	FWD: standard on Pinnacle

AWD: standard on Pinnacle

FWD/AWD: Packaged with S Appearance Package on Touring L and Limited

Type and material	Cast aluminum

Size	20 x 7.5

### Tires

Availability	Standard on Touring, Touring L

Size and type	235/65R17 BSW, all-season

Model	Yokohama Avid S34

Availability	Standard on Limited; Optional on Touring and Touring L AWD

Size and type	FWD: 235/60R18 BSW, all-season

AWD: 235/60R18 BSW all-season

FWD/AWD: 245/60/R18 BSW, all-season

Model	FWD 235 optional: Michelin Premier all-season self-sealing 
(Touring)

AWD 235 optional: Michelin Premier all-season self-sealing 
(Touring L AWD)

FWD/AWD 245 standard: Pirelli PZero self-sealing (Limited)

Availability	Standard on Pinnacle; optional on Touring L and Limited

Size and type	245/50R20 BSW, all-season

Model	FWD/AWD standard: Falken Ziex CT50

FWD/AWD optional: Pirelli PZero self-sealing (late availability)

###