# 2023 Dodge Challenger / Challenger SRT

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### General Information

Body Style	Two-door coupe, rear-wheel drive (RWD), all-wheel drive (AWD)

Assembly Plant	Brampton Assembly Plant, Ontario, Canada

EPA Vehicle Class	Intermediate car

Introduction Date	2014 as a 2015 model

### Body and CHASSIS

Layout	Longitudinal front engine, RWD or AWD

Construction	Unitized steel body

### Engine: 3.6-Liter PENTAStaR V-6

Availability	Standard — SXT RWD, SXT AWD, GT RWD and GT AWD

Type and Description	60-degree V-type, liquid-cooled

Displacement	220 cu. in. (3,604 cu. cm)

Bore x Stroke	3.78 x 3.27 (96.0 x 83.0)

Valve System	Chain-driven DOHC, 24 valves, hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block with aluminum-alloy heads

Compression Ratio	10.2:1

Power (SAE net)	303 bhp (227 kW) @ 6,350 rpm

Torque (SAE net)	268 lb.-ft. (363 N•m) @ 4,800 rpm

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane (R+M)/2

Oil Capacity	6 quarts (5.7 liters)

Coolant Capacity	14.0 quarts (13.25 liters)

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors and internal engine features

Exhaust Type	Dual exhaust with chromed tips

Max. Gross Trailer Weight	1,000 lbs. (454 kg)

### Engine: 3.6-Liter PENTAStaR V-6 (Continued)

EPA Fuel Economy mpg (city/hwy/combined)	19/30/23 (RWD); 18/27/21 (AWD)

Engine Assembly Plant	Trenton South Engine Plant, Trenton, Michigan

### ENGINE: 345 HEMI® V-8 WITH FUEL SAVER TECHNOLOGY

Availability	Standard — R/T models

Type and Description	90-degree V-type, liquid-cooled with variable-cam timing (VCT)

Displacement	345 cu. in. (5,654 cu. cm)

Bore x Stroke	3.92 x 3.58 (99.5 x 90.9)

Valve System	Pushrod-operated overhead valves, 16 valves, eight deactivating and eight conventional hydraulic lifters, all with roller followers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.5:1

Automatic Transmission

Power (SAE net)	372 hp (277 kW) @ 5,200 rpm

Torque (SAE net)	400 lb.-ft. (542 N•m) @ 4,400 rpm

Manual Transmission

Power (SAE net)	375 hp (280 kW) @ 5,150 rpm

Torque (SAE net)	410 lb.-ft. (555 N•m) @ 4,300 rpm

Max. Engine Speed	5,800 rpm (electronically limited)

Fuel Requirement	Unleaded midgrade, 89 octane (R+M)/2 — recommended – 
automatic transmission

Unleaded regular, 87 octane (R+M)/2 — acceptable – 
automatic transmission

Premium, 91 octane (R+M)/2 — recommended – manual

Oil Capacity	7 quarts (6.6 liters)

Coolant Capacity	14.7 quarts (13.9 liters)

Emission Controls 	Dual close-coupled three-way catalytic converters, 
quad heated oxygen sensors and internal engine features

Exhaust Type	Dual, 2.75-in. electronically controlled, performance-tuned active 
exhaust system

Max. Gross Trailer Weight	1,000 lbs. (454 kg) (not recommended with manual)

EPA Fuel Economy mpg (city/hwy/combined) 	16/25/19 (automatic); 15/23/18 (manual)

### ENGINE: 392 HEMI V-8 POWERED BY SRT

Availability	Standard — R/T Scat Pack models

Type and Description	90-degree V-type, liquid-cooled

Displacement	392 cu. in. (6,417 cu. cm)

Bore x Stroke	4.09 x 3.72 (103.9 x 94.6)

Valve System	Pushrod-operated overhead valves, 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic lifters, all with roller tips

Fuel Injection	Sequential, multiport, electronic, returnless; automatic features 
Fuel Saver mode

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.9:1

Power (SAE net)	485 bhp (362 kW) @ 6,100 rpm

Torque (SAE net)	475 lb.-ft. (644 N•m) @ 4,100 rpm

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity	7 quarts (6.6 liters)

Factory Oil Fill	0W-40 Pennzoil Ultra synthetic

Coolant Capacity	15 quarts (14.4 liters)

Emission Controls 	Dual close-coupled three-way catalytic converters, quad-heated oxygen sensors and internal engine features

Exhaust Type	Dual, 2.75-in. electronically controlled, performance-tuned active exhaust system

Max. Gross Trailer Weight	Not rated for trailer-tow

EPA Fuel Economy mpg (city/hwy/combined)	15/24/18 (automatic); 14/23/17 (manual)

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT V-8

Availability	Standard — SRT Hellcat

Type and Description	90-degree V-type, liquid-cooled

Displacement	376 cu. in. (6,166 cu.cm)

Bore x Stroke	4.09 x 3.58 (103.9 x 90.9)

Valve System	Pushrod-operated overhead valves, 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic lifters, all with roller tips

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, unique aluminum alloy heads with hemispherical combustion chambers

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT V-8 (CONTINUED)

Compression Ratio	9.5:1

Power (SAE net)	717 bhp (535 kW) @ 6,000 rpm

Torque (SAE net)	656 lb.-ft. (889 N•m) @ 4,800 rpm

Max. Engine Speed	6,200 rpm

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity	7 quarts (6.6 liters)

Factory Oil Fill	0W-40 Pennzoil Ultra synthetic

Coolant Capacity	15 quarts (14.4 liters)

Induction System 	Combined airflow rate of 1,048 CFM from:

Dual-snorkel hood

Air Catcher headlamp

Air box opening near wheel liner

Intercooler System	Separate low-temp cooling system with dual water-to-air intercoolers and a high-flow variable-speed electric water pump

When engine is shut down, radio-selectable Race Cooldown feature turns on radiator fan and intercooler water pump to reduce intercooler coolant temperature

Emission Controls 	Dual close-coupled three-way catalytic converters heated wide range upstream and switching downstream O2 sensors and internal engine features

Exhaust Type	Dual 2.75-in. straight-though exhaust system with twin electronic exhaust valves (EEV) with rectangular Black Vapor-chromed tips

Max. Gross Trailer Weight	Not rated for trailer-tow

EPA Fuel Economy mpg (city/hwy/combined)	13/22/16 (automatic); 13/21/16 (manual)

13/21/15 (automatic Widebody); 13/21/16 (manual Widebody)

Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT HIGH OUTPUT V-8

Availability	Standard — Challenger SRT Hellcat Redeye and SRT Super Stock

Type and Description	90-degree V-type, liquid-cooled

Displacement	376 cu. in. (6,166 cu. cm.)

Bore x Stroke	4.09 x 3.58 (103.9 x 90.9)

Valve System	Pushrod-operated overhead valve, 16 valves with sodium-filled exhausts and hollow stem intakes, 16 hydraulic roller lifters

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, 
aluminum alloy heads with hemispherical combustion chambers

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT HIGH OUTPUT V-8 (CONTINUED)

Compression Ratio	9.5:1

Power	797 bhp (595 kW) @ 6,300 rpm

807 bhp (601 Kw) @ 6,400 rpm on SRT Super Stock

Torque 	707 lb.-ft. (959 N•m) @ 4,500

Max. Engine Speed	6,500 rpm

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity	7 quarts (6.6 liters)

Factory Oil Fill	0W-40 Pennzoil Ultra synthetic

Coolant Capacity	15 quarts (14.4 liters)

Induction System 	Combined airflow rate of 1,130 CFM from:

Dual-snorkel hood

Air Catcher headlamp

Air box opening near wheel liner

Intercooler System	Separate low-temp cooling system with dual water-to-air intercoolers and a high-flow variable-speed electric water pump

SRT Power Chiller™ liquid-to-air intercooler chiller system is activated and redirects air conditioning refrigerant to chill the intercooler coolant (via radio button)

When engine is shut down, radio-selectable Race Cooldown feature turns on radiator fan and intercooler water pump to reduce intercooler coolant temperature

Emission Controls 	Dual close-coupled three-way catalytic converters heated wide-range upstream and switching downstream O2 sensors

Exhaust Type	Dual 2.75-in. straight-though exhaust system with twin electronic exhaust valves (EEV) with rectangular Black Vapor-chromed tips

Max. Gross Trailer Weight	Not rated for trailer-tow

EPA Fuel Economy mpg (city/hwy/combined)	13/22/16

13/21/15 (Widebody)

### Transmission: Six-speed Manual Tremec TR-6060 with ZF-Sachs 240 mm Twin Disc CLUTCH

Availability	Standard — Challenger R/T and R/T Scat Pack models

Description	Triple cone/double cone synchronizer design, 1-4 skip shift and reverse inhibit solenoids, 5:1 remote shifter

Gear Ratios

1st	2.97

2nd	2.10

3rd	1.46

4th	1.00

5th	0.74

6th	0.50

Reverse	2.90

Final Drive Ratio	3.90 — asymmetric limited-slip, performance-tuned

### TRANSMISSION: TREMEC SIX-SPEED MANUAL TR-6060 with ZF-SACHS 258 MM TWIN-DISC CLUTCH

Availability	Standard — Challenger SRT Hellcat

Description	Triple cone (1st, 2nd) / double cone (3rd-6th) synchronizer design; 
1-4 skip shift and Reverse inhibit solenoids; 5:1 remote shifter

Gear Ratios

1st	2.26

2nd	1.58

3rd	1.19

4th	1.00

5th	0.77

6th	0.63

Reverse	2.90

Final Drive Ratio	3.70 — asymmetric limited-slip, performance-tuned

Overall Top Gear	2.33

### TRANSMISSION: TORQUEFLITE 8HP50 EIGHT-SPEED AUTOMATIC

Availability	Standard — 3.6-liter Pentastar V-6 engine, RWD

Description	Adaptive electronic control, Sport mode and available paddle-shifted 
driver-interactive manual control. Five-clutch pack design with only two open clutches in any gear. Off-center line pump with low-viscosity oil for reduced spin loss. Torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

### TRANSMISSION: TORQUEFLITE 8HP50 EIGHT-SPEED AUTOMATIC (CONTINUED)

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.30

Final Drive Ratio	2.62 — standard

3.08 — optional

Overall Top Gear	1.76 — standard

2.06 — optional

### TRANSMISSION: TORQUEFLITE 850RE EIGHT-SPEED AUTOMATIC

Availability	Standard — 3.6-liter Pentastar V-6 engine, AWD

Description	Adaptive electronic control, Sport mode or paddle-shifted driver-interactive manual control. Five-clutch pack design with only two open clutches in any gear. Off-center line pump with low-viscosity oil for reduced spin loss. Torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.30

Final Drive Ratio	3.08

Overall Top Gear	2.06

### TRANSMISSION: TORQUEFLITE 8HP70 EIGHT-SPEED AUTOMATIC

Availability	Available with 5.7-liter and 392 HEMI V-8 engines

Description	Adaptive electronic control, configurable drive modes and paddle-shifted driver-interactive manual control, five-clutch pack design with only two open clutches in any gear, off-center line pump with low-viscosity oil for reduced spin loss, torque converter lock with turbine torsional damper for low lock-up speeds in first through eighth gear

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.11

4th	1.67

5th	1.29

6th 	1.00

7th 	0.84

8th 	0.67

Reverse	3.32

Final Drive Ratio	3.08 — standard with 5.7-liter

3.09 — asymmetric limited-slip, performance-tuned, 
standard with 392 HEMI V-8; optional with 5.7-liter

Overall Top Gear	2.06 (with 3.08 final drive ratio)

2.07 (with 3.09 final drive ratio)

### TRANSMISSION: TORQUEFLITE 8HP90 EIGHT-SPEED AUTOMATIC

Availability	Optional — SRT Hellcat; standard on SRT Hellcat Redeye and 
SRT Super Stock

Description	Adaptive electronic control with full manual control via gear selector 
or paddle shifters with three SRT-unique selectable modes: Street, 
Sport and Track (features performance shifting and gear holding)

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.10

4th	1.67

5th	1.29

6th	1.00

7th	0.84

8th 	0.67

Reverse	3.32

### TRANSMISSION: TORQUEFLITE 8HP90 EIGHT-SPEED AUTOMATIC (CONTINUED)

Final Drive Ratio	2.62 — asymmetric limited-slip, performance-tuned (standard)

3.09 — asymmetric limited-slip, performance-tuned 
(optional on SRT Hellcat Redeye, standard on SRT Super Stock)

Overall Top Gear	1.76 (with 2.62 final drive ratio)

2.07 (with 3.09 final drive ratio)

### TRANSFER CASE

Availability	Standard — AWD models

Type	Active, fully variable with front-axle disconnect

Center Differential	Planetary

Torque Split, Front/Rear	Fully variable

### Electrical System

Alternator	160-amp — standard on SXT, SXT AWD, GT AWD and R/T models

180-amp — optional on SXT, SXT AWD, GT AWD and R/T models

220-amp — standard on R/T Scat Pack, SRT Hellcat and 
SRT Hellcat Redeye models

Battery	H7 Case, 730 CCA maintenance-free

### Steering

Type	Electric power steering (EPS) with multi-mode assist

Overall Ratio	16.4:1 — SXT AWD and GT AWD models

14.4:1 — all RWD models

Turn Circle (curb-to-curb)	37.4 ft. — SXT and R/T models

37.8 ft. — R/T Scat Pack

38.7 ft. — SRT Hellcat models and Widebody models

Steering Turns (lock-to-lock)	2.6 — SXT and R/T models

3.1 — AWD models

2.6 — R/T Scat Pack models

2.5 — SRT Hellcat models

### Suspension

Front	Independent short- and long-arm (SLA) with high upper “A” arm

Lateral and diagonal lower links (RWD models), one-piece lower-control arms (AWD models)

Coil spring over gas-charged monotube shock absorbers

Bilstein Adaptive Damping Suspension (ADS) shock absorbers 
(available on R/T Scat Pack; standard on R/T Scat Pack 1320, 
R/T Scat Pack Widebody and SRT Hellcat models)

32 mm (34 mm for R/T Scat Pack Widebody and SRT models, 
28 mm included with 1320 Package) hollow stabilizer bar

Rear	Five-link independent with coil springs

Link-type 19 mm (22 mm on Widebody models) hollow stabilizer bar

Gas-charged monotube shock absorbers

Bilstein Adaptive Damping Suspension (ADS) shock absorbers (available on R/T Scat Pack; standard on R/T Scat Pack 1320, R/T Scat Pack Widebody and SRT Hellcat models)

Isolated suspension cradle

Sport Suspension 		Standard — SXT

Normal-duty Suspension	Standard — SXT AWD and GT AWD

Performance Suspension 	Standard — GT and R/T models

High-performance Suspension	Standard — R/T Scat Pack models

Available — GT and R/T models 
(included with Performance Handling Package)

SRT-tuned Three-mode ADS	Standard — R/T Scat Pack 1320:

Auto mode: sporty, compliant ride

Sport mode: firm, improved handling

Drag mode: optimized for weight transfer to rear

SRT-tuned Three-mode ADS	Standard — R/T Scat Pack Widebody and SRT Hellcat models; 
available on R/T Scat Pack:

Auto Mode: sporty, compliant ride

Sport Mode: firm, improved handling

Track Mode: firmest, maximum handling

SRT-tuned Three-mode ADS	Standard — SRT Super Stock:

Auto mode: sporty, compliant ride

Sport mode: firm, improved handling

Track mode: optimized for weight transfer to rear

### DIMENSIONS AND CAPACITIES

Wheelbase	116 (2,946)

Track, Front	63.4 (1,610)

64.0 (1,625) — SRT Hellcat models

65.6 (1,667) — R/T Scat Pack Widebody, SRT Hellcat Widebody models

Track, Rear	63.8 (1,620)

63.7 (1,618) — SRT Hellcat models

65.7 (1,618) — R/T Scat Pack Widebody, SRT Hellcat Widebody models

Overall Length	197.9 (5,027)

197.5 (5,017) — R/T Scat Pack Widebody, SRT Hellcat models

Overall Width (with mirrors)	85.4 (2,169)

Overall Width (without mirrors)	75.7 (1,923) (at widest point of vehicle, doors)

78.3 (1,990) — Widebody (at widest point of vehicle, front flares)

Overall Height	57.7 (1,465) — SXT, GT, R/T and R/T Scat Pack models

58.4 (1,482) — AWD models

57.2 (1,454) — SRT Hellcat models

57.5 (1,461) — SRT Hellcat Widebody

Ground Clearance	5.2 (130.9)

4.5 (115.2) — SRT Hellcat models

Drag Coefficient (Cd)	0.337 Cd — SXT

0.365 Cd — GT, R/T and R/T Scat Pack models

0.382 Cd — SRT Hellcat models

0.398 Cd — Widebody models

Frontal Area, sq. ft. (sq. m)	25.6 (2.4) — SXT

26.0 (2.4) — GT, R/T, R/T Scat Pack and SRT Hellcat models

26.3 (2.4) — AWD models

26.8 (2.5) — Widebody models

### DIMENSIONS AND CAPACITIES (CONTINUED)

Curb Weight (kg)	3,841 (1,742) — SXT

3,886 (1,762) — GT

4,051 (1,837) — SXT AWD

4,088 (1,854) — GT AWD

4,157 (1,885) — R/T (manual)

4,139 (1,877) — R/T (automatic)

4,124 (1,870) — R/T Scat Pack (with 1320 Package) without front passenger seat and rear seat

4,223 (1,915) — R/T Scat Pack (manual)

4,230 (1,918) — R/T Scat Pack (automatic)

4,298 (1,949) — R/T Scat Pack Widebody (manual)

4,303 (1,951) — R/T Scat Pack Widebody (automatic)

4,415 (2,002) — SRT Hellcat (manual)

4,422 (2,005) — SRT Hellcat (automatic)

4,446 (2,017) — SRT Hellcat Widebody (manual)

4,473 (2,029) — SRT Hellcat Widebody (automatic)

4,430 (2,009) — SRT Hellcat Redeye

4,481 (2,032) — SRT Hellcat Redeye Widebody

4,441 (2,014) — SRT Super Stock

Weight Distribution, percent F/R	52/48 — SXT

53/47 — GT

54/46 — SXT AWD, GT AWD and R/T (manual)

55/45 — R/T (automatic)

55/45 — R/T Scat Pack (manual)

55/45 — R/T Scat Pack (automatic)

57/43 — SRT Hellcat (manual)

57/43 — SRT Hellcat (automatic)

Fuel Tank capacity, gallons (liters)	18.5 (70.0)

### Accommodations

Seating Capacity — F/R	2/3 — standard

Front

Headroom	39.3 (999)

Knee clearance	4.4 (112)

Legroom	42.0 (1,067)

Shoulder room	58.5 (1,485)

Hip room	55.3 (1,404)

### Accommodations (CONTINUED)

Seat travel	Driver — 10.0 (255)

Passenger — 8.7 (220)

Recliner angle range	Driver — 64 degrees

Passenger — 63 degrees

EPA front volume index cu. ft. (cu. m)	55.8 (1.58)

Rear

Headroom	37.1 (942)

Legroom	33.1 (840)

Shoulder room	53.9 (1,368)

Hip room	47.8 (1,215)

EPA second row interior volume, cu. ft. (cu. m)	37.9 (1.07)

Total Interior Volume, cu. ft. (cu. m)	93.9 (2.67)

EPA Luggage Compartment Volume, cu. ft. (cu. m)	16.2 (0.459)

EPA Interior Volume Index, cu. ft. (cu. m)	110.1 (3.12)

Trunk Liftover Height	33.2 (845)

### TIRES

Availability	Standard on SXT

Size and type	235/55R18 All-season Performance

Mfr. and model	Pirelli P Zero

Revs per mile (km)	697 (433)

Availability	Standard on SXT AWD

Size and type	235/55R19 BSW All-season Performance

Mfr. and model	Pirelli P Zero

Revs per mile (km)	711 (442)

Availability	Optional on SXT (included with Plus Group and Blacktop Package)

Size and type	245/45ZR20 BSW All-season Performance

Mfr. and model	Firestone Firehawk GTV

Revs per mile (km)	726 (449)

Availability	Standard on GT, R/T and R/T Scat Pack models; optional on SXT AWD

Size and type	245/45ZR20 All-season Performance

Mfr. and model	Goodyear RS-A

Revs per mile (km)	733 (456)

### TIRES (CONTINUED)

Availability		Optional on GT, R/T and R/T Scat Pack models

Size and type	P245/45ZR20 BSW Performance

Mfr. and model	Goodyear Eagle F1

Revs per mile (km)	726 (451)

Availability	Standard on SRT Hellcat and SRT Hellcat Redeye; optional on 
R/T Scat Pack (included with T/A Package and Dynamics Package)

Size and type	275/40ZR20 All-season Performance

Mfr. and model	Pirelli P Zero

Revs per mile (km)	718 (446)

Availability		Optional on R/T (included with Performance Plus Group), 
R/T Scat Pack, SRT Hellcat and SRT Hellcat Redeye

Size and type	275/40ZR20 Summer

Mfr. and model	Pirelli P Zero

Revs per mile (km)	718 (446)

Availability	Optional on R/T Scat Pack (included with 1320 Package)

Size and type 	275/40R20 street-legal drag radials

Mfr. and model	Nexen N’FERA SUR4G Drag Spec

Revs per mile (km)	707 (439)

Availability	Optional on R/T Scat Pack, SRT Hellcat and SRT Hellcat Redeye 
(included with Widebody Package)

Size and type 	305/35ZR20 All-season Performance

Mfr. and model	Pirelli P Zero

Revs per mile (km)	731 (454)

Availability	Optional on R/T Scat Pack, SRT Hellcat and SRT Hellcat Redeye 
(optional with Widebody Package)

Size and type 	305/35ZR20 Three-season Performance

Mfr. and model	Pirelli P Zero

Revs per mile (km)	731 (454)

Availability	Standard on SRT Super Stock

Size and type 	315/40R18 102W

Mfr. and model	Nitto street-legal drag radials

Revs per mile (km)	742 (461)

### WHEELS

Availability	Standard on SXT

Type and material	Fine Silver / Aluminum

Size	18 x 7.5 in.

Availability	Standard on SXT AWD

Type and material	Satin Carbon / Aluminum

Size	19 x 7.5 in.

Availability	Optional on SXT AWD (included with Blacktop Package)

Type and material	Black Noise / Aluminum

Size	19 x 7.5 in.

Availability	Standard on GT

Type and material	Satin Carbon / Aluminum

Size	20 x 8 in.

Availability	Standard on GT AWD; optional on SXT (included with Plus Group)

Type and material	Satin Carbon / Aluminum

Size	20 x 8 in.

Availability	Standard on R/T, optional on GT (included with Plus Group)

Type and material	Machined with Granite pockets / Aluminum

Size	20 x 8 in.

Availability	Optional on SXT AWD and GT AWD (included with Blacktop Package)

Type and material	Black Noise / Aluminum

Size	20 x 8 in.

Availability	Standard on R/T; optional on SXT AWD (included with Plus Group)

Type and material	Satin Carbon / Aluminum

Size	20 x 8 in.

Availability	Optional on SXT, GT and R/T (included with Blacktop Package)

Type and material	Black Noise / Aluminum

Size	20 x 8 in.

Availability	Optional on GT and R/T (included with Performance Handling Group)

Type and material	Lights Out / Aluminum

Size	20 x 9 in.

Availability	Standard on R/T Scat Pack (included with Performance Handling Group and T/A Package)

Type and material	Low Gloss Granite / Aluminum

Size	20 x 9 in.

### WHEELS (CONTINUED)

Availability	Optional on R/T and R/T Scat Pack (included with Performance Plus Group, T/A Package, Dynamics Package and 1320 Package)

Type and material	Low Gloss Black / Aluminum

Size	20 x 9.5 in.

Availability		Optional on R/T Scat Pack (included with 1320 Package)

Type and material	Low Gloss Black with knurled bead seats / Aluminum

Size	20 x 9.5 in.

Availability	Standard on SRT Hellcat and SRT Hellcat Redeye models

Type and material	Low Gloss Black  / Aluminum

Size	20 x 9.5 in.

Availability	Optional on SRT Hellcat and SRT Hellcat Redeye models

Type and material	Machined with Granite pockets / Aluminum

Size	20 x 9.5 in.

Availability	Optional on R/T Scat Pack

Type and material	Brass Monkey / Aluminum

Size	20 x 9.5 in.

Availability	Standard on R/T Scat Pack (with Widebody Package)

Type and material	Low Gloss Granite “Devil’s Rim” / Aluminum

Size	20 x 11 in.

Availability	Standard on SRT Hellcat Redeye (with Widebody Package)

Type and material	Lightweight Low Gloss Black / Aluminum

Size	20 x 11 in.

Availability	Optional on SRT Hellcat, SRT Hellcat Redeye (with Widebody Package) and R/T Scat Pack (with T/A 392 Widebody Package); optional on R/T Scat Pack (with Widebody Package)

Type and material	Carbon Black / Aluminum

Size	20 x 11 in.

Availability	Optional on GT and R/T Scat Pack (with HEMI Orange Package)

Type and material	“Warp Speed” Carbon Black / Aluminum

Size	20 x 11 in.

Availability	Standard on SRT Super Stock

Type and material	Low Gloss Granite drag wheel / Aluminum

Size	18 x 11 in.

### Brakes

Sport Brake Package	Standard — SXT and SXT AWD

Front

Rotor size and type	12.6 x 1.1 (320 x 28) vented

Caliper size and type	2.36 (60) single-piston sliding with aluminum housing

Swept area	259 sq. in. (1,671 sq. cm)

Rear

Rotor size and type	12.6 x 0.4 (320 x 10) solid

Caliper size and type	1.65 (42) single-piston sliding with aluminum housing

Swept area	264 sq. in. (1,703 sq. cm)

Performance Brake Package	Standard — GT, GT AWD and R/T

Front

Rotor size and type	13.6 x 1.26 (345 x 28) vented

Caliper size and type	1.65 (42) dual-piston sliding with aluminum housing

Swept area	249 sq. in. (1,606 sq. cm)

Rear

Rotor size and type	12.6 x 0.87 (320 x 22) vented

Caliper size and type	1.65 (42) single-piston sliding with aluminum housing

Swept area	264 sq. in. (1,703 sq. cm)

Brembo High-performance Brake Package 	Standard — R/T Scat Pack and SRT Super Stock; optional — GT and 
R/T models

Front

Rotor size and type	14.2 x 1.26 (360 x 32.0) vented

Caliper size and type	1.73 (44) Brembo four-piston fixed monoblock aluminum

Swept area	351 sq. in. (2,261 sq. cm)

Rear

Rotor size and type	13.8 x 1.10 (350 x 28) vented

Caliper size and type	1.26 (32) Brembo four-piston fixed with aluminum housing

Swept area	316 sq. in. (2,036 sq. cm)

SRT/Brembo High-performance Brake Package 	Standard — SRT Hellcat models; optional — R/T Scat Pack models

Front

Rotor size and type	15.7 x 1.34 (400 x 34.0) two-piece aluminum hat, 
directional vane vented and slotted

Caliper size and type	1.42 (36 mm) Brembo six-piston fixed monoblock aluminum

Swept area	442 sq. in. (2,852 sq. cm)

### Brakes (continued)

Rear

Rotor size and type	13.8 x 1.10 (350 x 28) vented

Caliper size and type	1.26 (32) Brembo four-piston fixed with aluminum housing

Swept area	316 sq. in. (2,036 sq. cm)

Four-wheel Antilock Brake System (ABS) 
and Traction Control	Standard

Electronic Stability Control (ESC) 
and Brake Assist	Standard

Brake Knockback Mitigation	Standard

Hill-start Assist (HSA)	Standard

Three-mode (Normal, Sport, Off) 
ESC/Traction Control Mode Selection	Standard — GT, GT AWD and R/T models

Three-mode (Street, Drag, Off) 
ESC/Traction Control Mode Selection	Standard — R/T Scat Pack 1320

Four-mode (Street, Sport, Track, Off) 
ESC/Traction Control Mode Selection	Standard — R/T Scat Pack, SRT Hellcat and SRT Hellcat Redeye

Three-mode (Street, Track, Off) 
ESC/Traction Control Mode Selection	Standard — SRT Super Stock

Power-assist Type	8 + 9 (203 + 209) tandem-diaphragm vacuum booster

###