# 2023 Dodge Durango / Durango SRT

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### General Information

Vehicle Type	Four-door sport-utility vehicle

Assembly Plant	Detroit Assembly Complex — Jefferson, Detroit, Michigan

EPA Vehicle Class	Multipurpose vehicle

Introduction Date 	2010 as a 2011 model

### BODY AND CHASSIS

Layout	Longitudinal front engine, rear drive (transfer case with full-time 
all-wheel drive (AWD) available)

Construction	Steel unibody

### Engine: 3.6-Liter pentastar v-6

Availability	Standard — SXT, GT and Citadel (RWD and AWD)

Type and Description	60-degree V-type, liquid-cooled

Displacement	220 cu. in. (3,604 cu. cm)

Bore x Stroke	3.78 x 3.27 (96 x 83)

Valve System	Chain-driven DOHC, 24 valves and hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block, aluminum alloy heads

Compression Ratio	10.2:1

Power (SAE J2723)	293 hp @ 6,400 rpm – single exhaust

295 hp @ 6,400 rpm – dual exhaust

Torque (SAE J2723)	260 lb.-ft. (353 N•m) @ 4,000 rpm (both single and dual exhaust)

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane (R + M)/2

Oil Capacity	6 quarts (5.7 liters)

Coolant Capacity	10.4 quarts (9.9 liters) without trailer tow

11 quarts (10.4 liters) with trailer tow

### Engine: 3.6-Liter pentastar v-6 (continued)

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors and internal engine features

EPA Fuel Economy mpg (city/hwy/combined)	19/26/21 (RWD); 18/25/21 (AWD)

Engine Assembly Plant	Trenton South Engine Plant, Trenton, Michigan

### ENGINE: 5.7-Liter HEMI® V-8 WITH fUEL SAVER TECHNOLOGY

Availability	Standard — R/T and optional on Citadel (RWD and AWD)

Type and Description	90-degree V-type, liquid-cooled with variable-cam timing (VCT)

Displacement	345 cu. in. (5,654 cu. cm)

Bore x Stroke	3.92 x 3.58 (99.5 x 90.9)

Valve System	Variable-valve timing (VVT), pushrod-operated overhead valves, 
16 valves, eight deactivating and eight conventional hydraulic lifters, 
all with roller followers

Fuel Injection	Sequential, multi-port, electronic, returnless; Fuel Saver mode

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.5:1

Power (SAE J2723)	360 hp (268 kW) @ 5,150 rpm

Torque (SAE J2723)	390 lb.-ft. (529 N•m) @ 4,250 rpm

Max. Engine Speed	5,800 rpm (electronically limited)

Fuel Requirement	Unleaded mid-grade, 89 octane (R+M)/2 — recommended

Unleaded regular, 87 octane (R+M)/2 — acceptable

Oil Capacity	7 quarts (6.6 liters)

Coolant Capacity	15.4 quarts (14.6 liters) without trailer tow

16 quarts (15.2 liters) with trailer tow

Emission Controls 	Dual close-coupled three-way catalytic converters, quad heated oxygen sensors and internal engine features

EPA Fuel Economy mpg (city/hwy/combined)	14/22/17

Engine Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### ENGINE: 392 HEMI V-8 POWERED BY SRT WITH fUEL SAVER TECHNOLOGY

Availability	Standard — SRT 392

Type and Description	90-degree V-type, liquid-cooled

Displacement	392 cu. in. (6,417 cu. cm)

Bore x Stroke	4.09 x 3.72 (103.9 x 94.6)

### ENGINE: 392 HEMI V-8 POWERED BY SRT WITH fUEL SAVER TECHNOLOGY (continued)

Valve System	Pushrod-operated overhead valves, 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic lifters, all with roller tips

Fuel Injection	Sequential, multiport, electronic, returnless; Fuel Saver mode

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	10.9:1

Power (SAE J2723)	475 hp (354 kW) @ 6,000 rpm

Torque (SAE J2723)	470 lb.-ft. (637 N•m) @ 4,300 rpm

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity 	7 quarts (6.6 liters)

Coolant Capacity	14 quarts (13.25 liters)

Factory Oil Fill 	0W-40 Pennzoil Ultra Synthetic

Emission Controls 	Dual close-coupled three-way catalytic converters, 
quad-heated oxygen sensors and internal engine features

Exhaust Type 	Dual exhaust with 4-in. round with nickel-chrome tips

EPA Fuel Economy mpg (city/hwy/combined)	13/19/15

Engine Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT V-8

Availability	Standard — SRT Hellcat

Type and Description	90-degree V-type, liquid-cooled

Displacement	376 cu. in. (6,166 cu. cm)

Bore x Stroke	4.09 x 3.58 (103.9 x 90.9)

Valve System	Pushrod-operated overhead valves; 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic filters, all with roller tips

Fuel Injection	Sequential, multi-port, electronic, returnless; Fuel Saver mode

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio	9.5:1

Power (est.) (SAE J2723)	710 hp (529 kW) @ 6,100 rpm

Torque (est.) (SAE J2723)	645 lb.-ft. (637 N•m) @ 4,300 rpm

Max. Engine Speed	6,100 rpm (electronically limited)

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity	8.3 quarts (7.9 liters)

### ENGINE: SUPERCHARGED 6.2-LITER HEMI HELLCAT V-8 (continued)

Factory Oil Fill	OW-40 Pennzoil Ultra Synthetic

Coolant Capacity	14.7 quarts (13.9 liters)

Intercooler System		Separate low-temp cooling system with dual water-to-air intercoolers and a high-flow variable-speed electric water pump

Exhaust Type 	Dual exhaust with crossover X-pipe and 4-in. round nickel-chrome tips

EPA Fuel Economy mpg (city/hwy/combined)	TBD

Max. Gross Trailer Weight		8,700 lbs.

Engine Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### TRANSMISSION: 850RE torqueflite EIGHT-SPEED automatic

Availability	Standard with 3.6-liter Pentastar V-6 engine

Description	Driver-interactive manual control via steering wheel paddle shifters and electronically modulated torque converter clutch

Gear Ratios

1st 	4.71

2nd 	3.14

3rd  	2.11

4th 	1.67

5th 	1.29

6th	1.00

7th	0.84

8th	0.67

Reverse 	3.30

### TRANSMISSION: 8HP70 TORQUEFLITE AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard with 5.7-liter and 392 HEMI V-8 engines

Description	Driver-interactive manual control via steering wheel paddle shifters and electronically modulated torque converter clutch with Eco mode

Gear Ratios

1st	4.71

2nd 	3.14

3rd	2.11

4th 	1.67

5th 	1.29

6th	1.00

7th	0.84

### TRANSMISSION: 8HP70 TORQUEFLITE AUTOMATIC, EIGHT-SPEED OVERDRIVE (continued)

8th 	0.67

Reverse 	3.30

### TRANSMISSION: 8HP95 torqueflite AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard – SRT Hellcat

Description	Driver-interactive manual control via steering wheel paddle shifters and electronically modulated torque converter clutch

Gear Ratios

1st 	4.71

2nd 	3.14

3rd  	2.11

4th 	1.67

5th 	1.29

6th	1.00

7th	0.84

8th	0.67

Reverse 	3.30

### TRANSFER CASE: MP2010

Availability	Standard with V-6 engine (AWD)

Type	Single-speed

Operating Mode	Full-time AWD

Low Range Ratio	None

Torque Split, Front/Rear	50/50

### TRANSFER CASE: MP3025

Availability	Standard with 5.7-liter HEMI V-8 (AWD) with Tow Package

Type	Two-speed, electronically shifted

Operating Modes	AWD Low (Lock), Neutral; full-time active AWD

Low Range Ratio	2.72

Torque Split, Front/Rear	Variable

### TRANSFER CASE: MP3015

Availability	Optional with 5.7-liter HEMI V-8 (AWD)

Type	Single-speed, electronic proportioning

### TRANSFER CASE: MP3015 (continued)

Operating Modes	Full-time Active AWD

Low Range Ratio	None

Torque Split, Front/Rear	Variable

### TRANSFER CASE: MP3010

Availability	Standard with the 392 HEMI V-8 and 6.2-liter supercharged HEMI Hellcat V-8, available with 5.7-liter HEMI V-8 packaged with Tow N Go

Type	Single-speed, electronic proportioning

Operating Mode	Full-time AWD

Torque Split, Front/Rear	Variable:

Auto (40/60)

Snow and Tow (50/50)

Sport (35/65)

Track (30/70)

### Axles

Front

Differential Type	Conventional

Availability	Standard on AWD models

Ring Gear Diameter	7.7 (195)

Axle Ratios	3.45:1 (V-6 AWD models)

3.09:1 (5.7-liter HEMI V-8 models)

3.70:1 (392 HEMI V-8, 6.2-liter supercharged HEMI Hellcat V-8 models)

Rear

Differential Type	Conventional

Availability	Standard on all engines

Ring Gear Diameter	7.7 (195) (V-6 AWD models)

9.1 (230) (V-6 RWD and V-8 models)

Axle Ratios	3.45:1 (V-6 AWD models)

3.27:1 (V-6 RWD models)

3.09:1 (5.7-liter HEMI V-8 models)

3.70:1 (392 HEMI V-8, 6.2-liter supercharged HEMI Hellcat V-8 models)

### Electrical System

Alternator	160-amp; 180-amp or 220-amp depending on option content

Battery	Maintenance-free

H6 650 CCA + Auxiliary 200 CCA on electronic stop-start (V-6)

H7 700 CCA (V-8)

### STEERING

Overall Ratio	16.7:1 (V-6 AWD and all V-8 models)

17.9:1 (V-6 RWD)

Lock-to-Lock Steering Wheel Rotations	3.2 (V-6 AWD and all V-8 models)

3.4 (V-6 RWD)

Turning Diameter (curb-to-curb) 	41 ft. (12.5 m)

### Suspension (SXT, GT, R/T AND Citadel)

Front	Short- and long-arm (SLA) independent, coil springs, gas-charged, twin-tube coil-over shock absorbers, steel upper- and aluminum lower-control arms (“A” arms), aluminum knuckle, stabilizer bar

Rear	Multi-link rear suspension, coil springs, twin-tube shocks (including load leveling for towing), aluminum lower-control arm, independent upper links (tension and camber) plus a separate toe link

### Suspension (R/T Tow N Go, SRT 392 AND SRT HellCAT)

Front  	SRT-tuned SLA independent suspension with aluminum lower-control arms, aluminum knuckle, aluminum clevis, coil springs, Bilstein adaptive damping with rebound control springs, upper- and lower-control arms (“A” arms), hollow stabilizer bar

Rear 	SRT-tuned multi-link rear suspension, coil springs, Bilstein adaptive damping with rebound control springs, aluminum lower control arm, independent upper links (tension and camber) plus a separate toe link, hollow stabilizer bar

### Dimensions and Capacities

Wheelbase	119.8 (3,042)

Track, Front	63.9 (1,623) — SXT, GT, R/T and Citadel

64.5 (1,637) — SRT 392 and SRT Hellcat

Track, Rear	64.1 (1,627) — SXT, GT, R/T and Citadel

64.8 (1,645) — SRT 392 and SRT Hellcat

Overall Length	200.8 (5,101) — SXT, GT, R/T, Citadel and SRT 392

201 (5,104) — SRT Hellcat

Overall Width (Width at Mirrors)	85.5 (2,172)

### Dimensions and Capacities (continued)

Body Width	75.8 (1,924) — SXT, GT, R/T and Citadel

76.4 (1,940) — SRT 392 and SRT Hellcat

Overall Height (at antenna)	72.1 (1,831) — SXT RWD, GT RWD, R/T, Citadel RWD, SRT 392 
and SRT Hellcat

72.9 (1,851) — SXT AWD, GT AWD and Citadel AWD

Load Floor Height	32.2 (817) — SXT, GT, R/T and Citadel (AWD)

31.4 (798) — SXT, GT, R/T and Citadel (RWD), SRT 392 
and SRT Hellcat

Sill Step Height	19.7 (500) — SXT, GT, R/T, Citadel (RWD), SRT 392 and SRT Hellcat

20.5 (522) — SXT, GT, R/T and Citadel (AWD)

Ground Clearance	8.1 (207) — equipped with P265/60R18 tire and 3.6-liter engine

7.8 (200) — SRT 392 and SRT Hellcat

Chassis (fuel tank) 	10.0 (254)

Front Axle	9.6 (243.2)

Rear Axle	10.1 (256)

Approach Angle (degrees)	18.5 — SXT, GT, R/T and Citadel (RWD), SRT 392 and SRT Hellcat

20 — SXT, GT, R/T and Citadel (AWD)

Ramp Breakover Angle (degrees)	16.3 — SXT, GT, R/T and Citadel (RWD), SRT 392 and SRT Hellcat

18.1 — SXT, GT, R/T and Citadel (AWD)

Departure Angle (degrees)	21.5 — SXT, GT, R/T and Citadel (AWD)

20.2 — SXT, GT, R/T and Citadel (RWD), SRT 392 and SRT Hellcat

Frontal Area	31.8 sq. ft. (2.88 sq. m) — SRT 392

Drag Coefficient 	0.36 — SXT, GT, R/T and Citadel (RWD)

0.37 — SXT, GT, R/T and Citadel (AWD)

0.38 — SRT 392 and SRT Hellcat

Fuel Tank Capacity	24.6 gallons (93.1 liters)

### WEIGHTS (Estimates)

| DRIVE SYSTEM | MODEL | ENGINE | GVWR(a) lbs. (kg) | CURB WEIGHT(b) lbs. (kg) | PAYLOAD(c) lbs. (kg) |
| --- | --- | --- | --- | --- | --- |
| RWD | SXT | 3.6-liter | 6,500 (2,948) | 4,750 (2,155) | 1,400 (635) |
|  | GT | 3.6-liter | 6,500 (2,948) | 4,875 (2,212) | 1,440 (653) |
|  | Citadel | 3.6-liter 5.7-liter | 6,500 (2,948) 7,100 (3,221) | 4,990 (2,264) 5,215 (2,366) | 1,220 (553) 1,220 (553) |
|  | R/T | 5.7-liter | 7,100 (3,221) | 5,176 (2,349) | 1,350 (612) |
| AWD | SXT | 3.6-liter | 6,500 (2,948) | 4,884 (2,215) | 1,420 (644) |
|  | GT | 3.6-liter | 6,500 (2,948) | 4,986 (2,261) | 1,450 (658) |
|  | Citadel | 3.6-liter 5.7-liter | 6,500 (2,948) 7,100 (3,221) | 5,063 (2,298) 5,370 (2,437) | 1,220 (553) 1,220 (553) |
|  | R/T | 5.7-liter | 7,100 (3,221) | 5,313 (2,411) | 1,350 (612) |
|  | SRT 392 | 6.4-liter | 7,100 (3,221) | 5,378 (2,401) | 1,410 (640) |
|  | SRT Hellcat | 6.2-liter | 7,100 (3,221) | 5,575 (2,529) | 1,590 (721) |

(a) Gross Vehicle Weight Rating.

(b) Curb weight includes standard equipment and full quantities of fuel, lubricant and coolant.

(c) Payload is the maximum allowable weight of driver, passengers, cargo and options, rounded to the nearest 10 lbs. (5kg).

### Accommodations

Seating Capacity (front/second/third rows)	2/3/2 with third-row bench seat — SXT, GT and R/T

2/2/2 — Citadel, SRT 392 and SRT Hellcat, optional on GT and R/T

Front

Headroom	39.9 (1,013)

Legroom	40.3 (1,025)

Shoulder room	58.5 (1,486)

Hip room	57.0 (1,449)

Seat travel	11.0 (280) driver, 9.0 (230) passenger

SAE front volume index	54.4 cu. ft. (1.54 cu. m)

Second Row

Headroom	39.8 (1,011)

Legroom	38.6 (981)

Shoulder room	57.7 (1.465)

Hip room 	55.5 (1,410)

SAE volume	51.2 cu. ft. (1.45 cu. m)

Third Row

Headroom	37.8 (960)

### Accommodations (continued)

Legroom	33.5 (850)

Shoulder room	50.4 (1,281)

Hip room 	42.4 (1,077)

SAE volume	35.8 cu. ft. (1.01 cu. m)

SAE Cargo Volume

Behind third row	17.2 cu. ft. (0.49 cu. m)

Behind second row	43.3 cu. ft. (1.34 cu. m)

Behind front-row seats with rear seats folded	85.1 cu. ft. (2.41 cu. m)

### TIRES

Availability 	Standard on SXT

Size and type	P265/60R18

Mfr. and model 	Michelin LTX All-season

Availability 	Standard on GT, R/T and Citadel; optional on SXT (included with Blacktop and Anodized Platinum packages)

Size and type 	P265/50R20

Mfr. and model 	Bridgestone Ecopia H/L 422 Plus Black Sidewall All-season

Availability 	Optional on GT and R/T (included with Blacktop Package)

Size and type 	P265/50R20

Mfr. and model 	Pirelli Scorpion Verde Black Sidewall All-season Performance

Availability	Standard on SRT 392 and SRT Hellcat; optional on R/T Tow N Go

Size and type	P295/45ZR20

Mfr. and model	Pirelli Scorpion Zero — All-season Performance (run flat)

Revs per mile	699

Availability	Optional on SRT 392 and SRT Hellcat

Size and type 	P295/45ZR20

Mfr. and model	Pirelli P-Zero — Three-season (run flat)

Revs per mile	699

### Wheels

Availability	Standard on SXT

Type and material	Painted Tech Silver, aluminum

Size (inches)	18 x 8 in.

### Wheels (continued)

Availability	Standard on GT and R/T

Type and material	Painted Fine Silver, aluminum

Size (inches)	20 x 8 in.

Availability	Standard on Citadel; optional on SXT (included with Anodized Platinum Package)

Type and material	Satin Carbon, aluminum

Size (inches)	20 x 8 in.

Availability	Optional on SXT (with Plus Group), GT and R/T (all packaged with Blacktop)

Type and material	Black Noise, aluminum

Size (inches)	20 x 8 in.

Availability	Standard on SRT 392; optional on R/T (packaged with Tow ‘n Go)

Type and material	Hyper Black, aluminum

Size (inches)	20 x 10 in.

Availability	Optional on R/T (packaged with Tow N Go and Blacktop), SRT 392 and SRT Hellcat (packaged with SRT Black)

Type and material	Painted Lights Out, aluminum

Size (inches)	20 x 10

Availability	Standard on SRT Hellcat

Type and material	Machined face, Mid-gloss Black pockets

Size (inches)	20 x 10

### BRAKES

Type	Power-assisted, four-wheel antilock brake system (ABS)

Front

Rotor size and type Inches (mm)	13.8 x 1.26 (350 x 32) vented disc

15.0 x 1.34 (380 x 34) vented disc with Brembo six-piston calipers — SRT 392, available on R/T (packaged with Tow N Go)

15.0 X 1.34 (480 x 34) two-piece aluminum performance rotor hat, slotted with Brembo six-piston caliber and ABS — Optional on SRT 392

15.7 X 1.42 (400 x 36) two-piece aluminum performance rotor hat, slotted with Brembo six-piston caliber and ABS — SRT Hellcat

Caliper size and type Inches (mm)	1.89 (48) two-piston pin-slider caliper

1.57 (40) Brembo six-piston fixed caliper — SRT 392 and SRT Hellcat, available on R/T (packaged with Tow N Go)

Swept area (sq. in. / sq. cm)	324 (2,088)

413 (2,666) — SRT 392, available on R/T (packaged with Tow N Go)

498 (3,215) — SRT Hellcat

### BRAKES (continued)

Rear

Rotor size and type	13.0 x 0.87 (330 x 22) vented disc

13.78 x 1.10 (350 x 28) disc — SRT 392 and SRT Hellcat, available on R/T (packaged with Tow N Go)

Caliper size and type	1.89 (48) single-piston floating caliper

1.34 (34) Brembo four-piston fixed caliper — SRT 392 and SRT Hellcat, available on R/T (packaged with Tow N Go)

Swept area (sq. in. / sq. cm)	257 (1,658)

316 (2,039) — SRT 392 and SRT Hellcat, available on R/T (packaged with Tow N Go)

Parking Brake Type	Rear, mechanical, drum-in-hat

Power Assist	Single-rate, tandem diaphragm vacuum

Four-wheel Antilock Brakes	Standard

Electronic Stability Control	Standard

All-speed Traction Control	Standard

Brake Assist	Standard

### TRAILER TOWING: must have optional towing package to achieve maximum towing capacity

| DRIVE SYSTEM | ENGINE | AXLE RATIO | MAXIMUM TRAILER WEIGHT lbs. (kg) |
| --- | --- | --- | --- |
| RWD | 3.6-liter V-6 5.7-liter V-8 | 3.27 3.09 | 6,200 (2,812) 7,400 (3,356) |
| AWD | 3.6-liter V-6 5.7-liter V-8 5.7-liter V-8 (packaged with Tow N Go) 6.4-liter and 6.2-liter V-8 | 3.45 3.09 3.09 3.70 | 6,200 (2,812) 7,200 (3,266) 8,700 (3,946) 8,700 (3,946) |

###