# 2023 FIAT 500X

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Body Style 	Crossover

Assembly Plant	Melfi, Italy

EPA Vehicle Class 	Multipurpose vehicle

### BODY AND CHASSIS

Layout	Transverse front engine, AWD

Construction 	Steel uniframe

### ENGINE: 1.3-LITER TURBOCHARGED

Availability	Standard — Pop AWD, Sport AWD

Type and Description	In-line four-cylinder, liquid-cooled, turbocharged

Displacement	1,332 cu. in. (cc)

Bore x Stroke	70 mm x 86.5 mm

Valve System	SOHC and hydraulic valve actuation

Fuel Injection	Direct injection

Construction	Aluminum block and aluminum heads

Compression Ratio	10.5

Maximum Turbo Boost (psi / bar)	39 psi / 2.7 bar (absolute)

Power (SAE)	177 hp (132 kW) @ 5,500 rpm

Torque (SAE)	210 lb.-ft. (285 N•m) @ 2,200 rpm

Max. Engine Speed	6,500 rpm (electronically limited)

Fuel Requirement	87 octane (R+M)/2 acceptable

91 octane recommended

Oil Capacity	4.5 quarts (4.3 liters) with dry filter

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors 
and internal engine features

Fuel Economy (city/hwy/combined)	24/30/26

Towing (lb.)	2,000

Engine Assembly Plant	Bielsko-Biała, Poland

### TRANSMISSION: NINE-SPEED AUTOMATIC

Availability	1.3-liter turbocharged engine (AWD)

Description	Planetary gear train, transverse layout

Ratio Spread	9.81

Gear Ratios

1st	4.71

2nd	2.84

3rd	1.91

4th	1.38

5th	1.00

6th	0.81

7th	0.70

8th	0.58

9th	0.48

Reverse	3.83

Axle Ratio	3.734

### ELECTRICAL SYSTEM

Alternator 	150 amp

Battery	500 amp

### SUSPENSION

Front	MacPherson strut, coil springs, high-strength steel front cross member, high-strength steel mono shell lower control arms, stabilizer bar

Rear	Chapman strut, high-strength steel links, isolated steel rear cradle, 
coil springs, stabilizer bar

### STEERING

Type	Electric power rack and pinion

Overall Ratio 	15.7

Turning Diameter (curb-to-curb) 	36.3 (11.07)

Steering Turns (lock-to-lock)	2.68

### BRAKES

Front

Size and type	12 x 1.1 (305 x 28) vented rotor with 
2.36 (60) single-piston floating caliper

Swept area (per caliper)	138.77 sq. in. (895.3 sq. cm), 1 x 60

Rear

Size and type 	10.95 x 0.47 (278 x 12) solid rotor with 
1.5 (38) single-piston floating caliper

Swept area (per caliper)	101.7 sq. in. (656.4 sq. cm), 1 x 38

Power-assist Type 	10 (254) vacuum assist

Four-wheel anti-lock brake system (ABS)	Standard

Electronic stability control (ESC)	Standard

Parking Brake Type	Electric motor on caliper

### DIMENSIONS AND CAPACITIES

Wheelbase	101.2 (2,570)

Track, Front	60.7 (1,543)

Track, Rear	60.7 (1,543)

Overall Length	167.2 (4,248) Pop, Sport

Overall Width	73.2 (1,859) with mirrors folded

79.7 (2,025)

70.7 (1,796) without mirrors

Overall Height	63.7 (1,617)

Load-floor Height	29.8 (758)

Sill-step Height	19.4 (494) front, 19.6 (497) rear

Ground Clearance	7.9 (202)

Curb Weight, lb. (kg)	3,305 (1,499)

Aero Cd	0.350

Fuel-tank Capacity, gallons (liters)	12.7 (48)

### ACCOMMODATIONS

Seating Capacity — F/R	2/3

Front Row

Headroom	39.1 (993)

Legroom	41.4 (1,053)

Shoulder room 	54.3 (1,380)

Hip room	53.3 (1,354)

Seat travel 	9.0 (230) driver / 10.2 (260) front passenger

Recliner range (degrees)	From 25 nominal, 38 forward and 30 rearward 
(from vertical 13 forward, 55 rearward)

First row interior volume 
(standard roof), cu. ft. (cu. m)	51.5 cu. ft (1.46 cu. m)

Second Row

Headroom 	37.8 (961)

Legroom 	34.8 (883)

Shoulder room	52.8 (1,342)

Hip room	52.2 (1,326)

Second row interior volume, cu. ft. (cu. m)	40.2 cu. ft. (1.14 cu. m)

Cargo Access

Liftover height	31.4 (797)

Maximum cargo width at liftgate opening	41.3 (1,048)

Minimum cargo width at liftgate opening	37.1 (943) (at floor)

Cargo height 	28.8 (731)

Rear opening height 	26.6 (676)

Distance between wheelhouse interior trim	37.8 (960)

SAE Cargo Volume

Rear seats up, cu. ft. (cu. m) 	14.1 (0.40)

Rear seats folded, cu. ft. (cu. m) 	39.8 (1.127)

Rear seats folded (to headliner), cu. ft. (cu. m) 	32.1 (0.91)

Consumer Reports cargo, cu. ft. (cu. m)	20.0 (0.566)

### TIRES

Availability	Standard — Pop

Size and type 	215/60R17 BSW all-season

Mfr. and model 	Continental

### TIRES (CONTINUED)

Availability	Standard — Sport

Optional — Pop

Size and type 	225/55R18 BSW all-season

Mfr. and model	Continental

Availability	Optional — Sport

Size and type 	225/45R19 all-season

Mfr. and model 	Michelin

###