# 2023 Jeep® Compass

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Body Style	Sport-utility vehicle (SUV)

Assembly Plant	Toluca, Mexico

EPA Vehicle Class	Multipurpose vehicle

### BODY AND CHASSIS

Layout	Transverse front engine, 4x4 only

Construction	Steel uniframe

### ENGINE: 2.0-LITER TURBO I-4 ENGINE WITH ENGINE STOP-START (ESS)

Availability	Standard on all models

Type and Description	Inline Turbo DOHC

Displacement	121.7 cu. in. (1995 cc)

Bore x Stroke	3.31 x 3.54 (84.0 x 90)

Valve System	Chain-driven, DOHC, variable-valve timing for intake and exhaust

Fuel Injection	Direct injection

Construction	Aluminum block and aluminum heads

Compression Ratio	10.0:1

Power (SAE net)	200 hp (149.14 kW) @ 5,000 rpm (100 hp/liter)

Torque (SAE net)	221 lb.-ft. (300 N•m) @ 1,750-4,250 rpm

Max. Engine Speed	5,800 rpm, electronically limited

Fuel Requirement	Regular unleaded, 87 octane (R+M)/2

Oil Capacity	5.0 quarts (4.7 liters)

Coolant Capacity	3.4 quarts (3.25 liters)

Emission Controls	Cooled, high-pressure EGR. Close coupled three-way catalytic converters. Engine stop-start (ESS) technology

EPA Fuel Economy mpg (city/hwy/combined)	24/32/27*

Towing	Up to 2,000-lb. (when properly equipped)

Engine Assembly Plant	Kokomo Engine Plant, Kokomo IN

*Estimated mpg

### TRANSMISSION: 8F30 eight-SPEED AUTOMATIC

Availability	2.0-liter Turbo I-4 engine with ESS (4x4)

Description	Planetary gear train, transverse layout

Ratio Spread	7.43

Gear Ratios

1st	4.72

2nd	2.91

3rd	1.86

4th	1.42

5th	1.22

6th	1.00

7th	0.79

8th	0.64

Reverse	3.24

Final Drive Ratio	3.73 AWD / 4.398 Trailhawk

### 4X4 SYSTEMS: JEEP ACTIVE DRIVE, JEEP ACTIVE DRIVE LOW

Availability 	Jeep Active Drive is standard on all models

Jeep Active Drive Low is standard on Trailhawk

Type	Full-time 4x4 mode with active on-demand clutch

### 4X4 SYSTEMS: JEEP ACTIVE DRIVE, JEEP ACTIVE DRIVE LOW

Operating Modes	Auto 4x4, neutral

Center Differential Type	None

Terrain Response	Unique tuning in all terrain modes

Crawl Ratio	20:1 (with Jeep Active Drive Low)

### ELECTRICAL SYSTEM

Alternator	180-amp

Battery	Dual batteries with electronic stop-start system: 650-amp (primary), 200-amp (secondary)

### SUSPENSION

Front 	MacPherson strut, coil springs, flat front steel crossmember, 
high-strength steel double shell lower control stabilizer bar

Rear	Chapman strut, high-strength steel links, isolated steel rear cradle, 
coil springs, stabilizer bar

### STEERING

Type 	Electric power rack and pinion

Overall Ratio	15.7

Turning Diameter (curb-to-curb)	36.3 (11.07)

35.3 (10.76) Trailhawk 4x4

Steering Turns (lock-to-lock)	2.68

2.76 (Trailhawk 4x4)

### BRAKES

Front

Size and type	12 x 1.1 (305 x 28) vented rotor with 2.36 (60) single-piston floating caliper

Swept area (per caliper)	138.77 sq. in. (895.3 sq. cm), 1 x 60

115.88 sq. in. (747.6 sq. cm), 1 x 57

Rear

Size and type	10.95 x 0.47 (278 x 12) solid rotor with 1.5 (38) single-piston floating caliper

Swept area (per caliper)	101.7 sq. in. (656.4 sq. cm), 1 x 38

Power-assist type	10 (254) vacuum assist

Four-wheel antilock brake system (ABS)	Standard

Electronic stability control (ESC)	Standard

Parking brake type	Electric motor

### DIMENSIONS AND CAPACITIES

Wheelbase	103.8 (2,636)

Track, Front	60.7 (1,542)

Track, Rear	60.3 (1,532)

Overall Length	173.4 (4,404)

Overall Width (with folded side mirrors)	73.8 (1,874)

Overall Width (with open side mirrors)	80 (2,033)

Overall Height	64.6 (1,641) without rails; 64.8 (1,647) with rails

Load-floor Height	29.7 (754) (height measured from curb ground to load floor)

Sill-step Height	19.4 (494) (height measured from curb ground to sill trim)

Ground Clearances	4x4 8.1 (207)

Trailhawk 4x4 8.6 (219)

### DIMENSIONS AND CAPACITIES (continued)

Approach Angle (with air dam) (degrees)	4x4 16.1

Trailhawk 4x4 30.4

Breakover Angle (degrees)	4x4 22.5

Trailhawk 4x4 23.6

Departure Angle (degrees)	4x4 31.4

Trailhawk 4x4 34.0

Curb Weight, lbs. (kg)	3,620 lbs. (1,642)

Fuel Tank Capacity	13.5 gal. (51 liters)

### ACCOMMODATIONS

Seating Capacity — Front/Rear	2/3

Front Row

Headroom without sunroof	39.2 (995)

Headroom with sunroof	38.6 (981)

Legroom	41.2 (1,046)

Shoulder room	56.7 (1,439)

Hip room	54.1 (1,375)

Seat travel	8.7 (220) fore/aft travel range

Recliner range (degrees)	Manual/power recliner travel: 69 degrees

Manual fold FWD recliner travel: 107.5 degrees

SAE volume (standard roof), cu. ft. (cu. m)	52.9 (1.49) front passenger volume

Second Row

Headroom	38.5 (978)

Legroom	38.3 (973)

Shoulder room	55.1 (1,400)

Hip room	49.2 (1,250)

SAE volume, cu. ft. (cu. m)	46.7 (1.33) second-row passenger volume

Cargo Access

Liftover height	31.1 (792) (height measured from curb ground to liftgate scuff trim)

Maximum cargo width at liftgate opening	42.1 (1,069)

Minimum cargo width at liftgate opening	36.9 (938)

Maximum cargo height at liftgate opening	27.2 (691)

Distance between wheelhouse interior trim	38.1 (968)

### ACCOMMODATIONS (continued)

SAE Cargo Volume

Rear seats up, cu. ft. (cu. m)	27.2 (0.8)

Rear seats folded, cu. ft. (cu. m)	59.8 (1.7)

EPA interior volume index, cu. ft. (cu. m)	126.7 (3.6)

### TIRES

Availability	Standard — Sport 4x4 and Latitude 4x4

Size and type	225/60R17 BSW All-season

Mfr. and model	Firestone Destination LE2

Revs per mile (km)	760 (472)

Availability	Standard — Altitude 4x4, Limited 4x4

Size and type	225/55R18 BSW All-season

Mfr. and model	Continental Pro Contact

Revs per mile (km)	744 (462)

Availability	Standard — Trailhawk 4x4

Size and type	215/65R17 on/off-road

Mfr. and model	Falken WildPeak H/T

Revs per mile (km)	745 (463)

Availability	Standard — Latitude LUX 4x4

Size and type	225/55R18 BSW All-season

Mfr. and model	Yokohama Geolandar

Revs per mile (km)	752 (467)

Availability	Standard — High Altitude 4x4 and RED Edition 
(optional on Limited 4x4)

Size and type	235/45R19 BSW All-season

Mfr. and model	Continental Pro Contact

Revs per mile (km)	752 (467)

###