# 2023 Jeep® Gladiator

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### General Information

Vehicle Type	Midsize truck

Assembly Plant	Toledo Supplier Park, Toledo, Ohio

EPA Vehicle Class	Midsize truck

Introduction Date	2019

### BODY AND CHASSIS

Layout	Longitudinal front engine, four-wheel drive

Construction	Ladder-type frame, open steel and aluminum body

### Engine: 3.6-LITER PENTASTAR V-6

Availability	Standard — all models

Type and Description	60-degree, V-type, liquid-cooled

Displacement (cu. in. / cc)	220 cu. in. (3,604 cu. cm)

Bore x Stroke 	3.78 x 3.27 (96 x 83)

Valve System	Chain-driven DOHC, 24 valves and hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block, aluminum alloy heads

Compression Ratio	11.3:1

Power (SAE net) (hp / kW@rpm)	285 hp (209 kW) at 6,400 rpm

Torque (SAE net) (lb.-ft. / N•m)	260 lb.-ft. (353 N•m) at 4,400 rpm

Max. Engine Speed (rpm)	6,600 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane (R + M)/2

Fuel Tank Capacity (gallons)	22

Oil Capacity 	5.7 quarts (5.4 liters)

Coolant Capacity	10 quarts (10.1 liters)

Emission Controls	Dual three-way catalytic converters, heated oxygen sensors and internal engine features(a)

Max. Gross Trailer Weight	7,650 lbs.

### Engine: 3.6-LITER PENTASTAR V-6 (CONTINUED)

EPA Fuel Economy mpg (city/hwy/combined)	17/22/19 — automatic

16/23/19 — manual

Assembly Plant	Saltillo South Engine Plant, Saltillo, Coahuila, Mexico

### ENGINE: 3.0-LITER ECODIESEL V-6

Availability	Available — Sport, Rubicon, Overland

Type and Description	60-degree diesel V-6

Displacement	182 cu. in. (2,987 cu. cm)

Bore x Stroke	3.27 x 3.60 (83 x 92)

Valve System	Chain driven DOHC, 24 valves, hydraulic lash adjusters 
with roller fingers follower

Fuel Injection	Common rail, 29,000 psi (2000 bar), solenoid injectors

Construction	Compacted graphite iron cylinder block, aluminum alloy heads

Compression Ratio	16.0:1

Power	260 hp (194 kW) @ 3,600 rpm

Torque	442 lb.-ft. (600 N•m) @ 1,400-2,800 rpm

Max. Engine Speed	4,600 rpm

Fuel Requirement	Ultra-low sulfur diesel

Fuel Tank Capacity (gallons)	18.3 (4-door)

Oil Capacity	9.0 quarts (8.5 liters)

Coolant Capacity	12.3 quarts (11.6 liters)

Emission Controls	High-pressure EGR, low-pressure EGR, oxidation catalyst, 
diesel particulate filter, SCR with urea injection

Max. Gross Trailer Weight	6,500 lbs.

EPA Fuel Economy mpg (city/hwy)	22/28/24 — Sport and Overland

20/25/22 — Rubicon

Assembly Plant	Cento, Italy

### Transmission: 850RE automatic, Eight-speed overdrive

Availability	Standard — Overland (3.6-liter V-6), Optional — Sport, Rubicon, Mojave (all with 3.6-liter V-6)

Description	Adaptive electronic control or Electronic Range Select (ERS) 
driver-interactive manual control and electronically modulated 
torque converter clutch

### Transmission: 850RE automatic, Eight-speed overdrive (CONTINUED)

Gear Ratios

1st	4.71

2nd	3.13

3rd	2.10

4th	1.67

5th	1.28

6th	1.00

7th	0.84

8th	0.67

Reverse	3.30

### TRANSMISSION: D478 MANUAL, SIX-SPEED WITH OVERDRIVE

Availability	Standard — Sport, Rubicon, Mojave (all with 3.6-liter V-6)

Description	Synchronized in all forward gears and reverse; multi-rail shift system 
with top-mounted shift lever

Gear Ratios

1st	5.13

2nd	2.63

3rd	1.53

4th	1.00

5th	0.81

6th	0.72

Reverse	4.49

Final Drive Ratio	3.73 (Sport, Overland), 4.10 (Rubicon, Mojave, Sport Max. Tow)

### TRANSMISSION: 8HP75 AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard — 3.0-liter EcoDiesel V-6

Description	Adaptive electronic control or Electronic Range Select (ERS) 
driver-interactive manual control and electronically modulated 
torque converter clutch

### TRANSMISSION: 8HP75 AUTOMATIC, EIGHT-SPEED OVERDRIVE (CONTINUED)

Gear Ratios

1st	4.71

2nd	3.13

3rd	2.10

4th	1.67

5th	1.28

6th	1.00

7th	0,84

8th	0.67

Reverse	3.30

### TRANSFER CASE: NV241 command-trac (MP1622)

Available	Standard — Sport, Overland, Mojave

Type	Part-time

Operating Modes	2WD High; 4WD High; Neutral; 4WD Low

Low Range Ratio	2.72:1

Axle Ratio	3.73 / 4.10 (Mojave, Sport Max. Tow)

### TRANSFER CASE: MP3022 Selec-Trac

Available	Optional — Sport, Overland, Mojave

Type	Full-time

Operating Modes	2WD High; Auto; 4WD High; Neutral; 4WD Low

Low Range Ratio	4.0:1

Axle Ratio	3.73 / 4.10 (Mojave, Sport Max. Tow)

### TRANSFER CASE: NV241OR Rock-trac (MP1622OR)

Available	Standard — Rubicon

Type	Standard — Part-time
Available — Full-time

Operating Modes	2WD High; 4WD High; Neutral; 4WD Low

Low Range Ratio	4.0:1

Axle Ratio	4.10

### Axles

Front	3rd generation Dana heavy-duty axles

Differential type	Open (Sport, Sport S, Overland, Mojave) or Tru-Lok electronic locking (Rubicon)

Axle ratios	3.73, 4.10

Rear	3rd generation Dana heavy-duty axles

Differential type	Open (Sport, Overland) with available Trac-Lok anti-spin

Tru-Lok electronic locking (Mojave, Rubicon)

Axle ratios	3.73, 4.10 (standard on Sport Max. Tow, Mojave, Rubicon)

### Electrical System

Alternator (standard)	180 amp (standard Sport), 220 amp (standard all models except Sport) or 240 amp (Trailer Tow, Aux switches, Max Tow, Cargo Management)

Battery	H6 650 CCA (standard) or H7 700 CCA (Aux switches)

### Suspension

Front	Solid axle, link coil, leading arms, track bar, coil springs, stabilizer bar, FOX hydraulic jounce bumper (Mojave exclusive)

Stabilizer bar	Electronic Sway-bar Disconnect System — standard, Rubicon

Shock type	Sport and Overland: Gas-charged monotube shock absorbers with Multi-tuned Valve (MTV) technology

Mojave: FOX 2.5-in. diameter aluminum internal bypass shock absorbers with external reservoirs

Rubicon: High-pressure gas-charged monotube shock absorbers with hydraulic rebound stop

Rear	Solid axle, link coil, trailing arms, track bar, coil springs, stabilizer bar

Shock type	Sport and Overland: Gas-charged monotube shock absorbers with Multi-tuned Valve (MTV) technology

Mojave: FOX 2.5-in. diameter aluminum internal bypass shock absorbers with external reservoirs

Rubicon: High-pressure, gas-charged monotube shock absorbers with hydraulic rebound stop

### STEERING

Type	Electro-hydraulic power

Overall ratio	13.3:1

Turning diameter (curb-to-curb) (ft./m)	44.5 — Sport, Overland, 44.8 — Rubicon, 45.0 — Mojave

Steering turns (lock-to-lock)	3.24

P/S Pump	Electro-hydraulic

### BRAKES

Type	Power-assisted, antilock braking system

Availability	Standard

Front

Rotor size and type	12.9 x 1.1 (330 x 28) vented rotor

Caliper size and type	2 (51) twin-piston floating calipers

Rear

Rotor size and type	13.6 x .86 (345 x 22) vented rotor

Caliper size and type	2 (51) single-piston floating calipers

### Dimensions and Capacities

Wheelbase	137.3 (3,487)

Overhang — Front	29.7 (755)

Overhang — Rear	51.0 (1,296)

Track — Front	62.9 (1,598) Sport, Overland / 64.4 (1,636) Sport Max. Tow, 
Rubicon / 65 (1,650) Mojave

Track — Rear	62.9 (1,598) Sport, Overland / 64.4 (1,636) Sport Max. Tow, 
Rubicon / 65 (1,650) Mojave

Overall Length	218.0 (5,539)

Overall Width	73.8 (1,875)

Overall Height — Soft-top	75.0 (1,907) / 76.1 (1,933) — Rubicon

Overall Height — Hardtop	73.1 (1,857) / 74.1 (1,882) — Rubicon

Box Volume	35.5 cu. ft. (1.0 cu. m)

Box Length, Tailgate Closed	60.3 (1,531)

Box Length, Tailgate Opened	81.3 (2,067)

Cargo Width — Liftgate	56.8 (1,442)

Cargo Height — Liftgate	33.9 (861)

Liftover Height	29.7 (755)

### Dimensions and Capacities (CONTINUED)

Load Floor Height — Axle	33.3 (845)

Distances Between Wheelhouses	44.8 (1,137)

Tailgate Opening Width	50 (1,270)

Open Tailgate to Ground	34.8 (885)

### CLEARANCES

| Clearances | 245/75R17 Standard on Sport | 255/70R18 Standard on Overland | LT285/70R17 Standard on Mojave | LT285/70R17 Standard on Rubicon |
| --- | --- | --- | --- | --- |
| Approach Angle (degrees) | 40.8 | 40.8 | 44.7 | 43.4 |
| Breakover Angle (degrees) | 18.4 | 18.4 | 20.9 | 20.3 |
| Departure Angle (degrees) | 25 | 25 | 25.5 | 26 |
| Ground Clearance (inches) | 10 (253) | 10 (253) | 11.6 | 11.1 (283) |

### CURB WEIGHT

Model		3.6-liter Pentastar V-6

Jeep Gladiator Sport

Automatic transmission (lbs./kg)	4,672 (2,119)

Manual transmission (lbs./kg)	4,650 (2,109)

Jeep Gladiator Sport S

Automatic transmission (lbs./kg)	4,672 (2,119)

Manual transmission (lbs./kg)	4,650 (2,109)

Jeep Gladiator Overland

Automatic transmission (lbs./kg)	4,742 (2,137)

Manual transmission (lbs./kg)	4,720 (2,141)

Jeep Gladiator Mojave

Automatic transmission (lbs./kg)	4,974 (2,256)

Manual transmission (lbs./kg)	4,952 (2,246)

Jeep Gladiator Rubicon

Automatic transmission (lbs./kg)	5,072 (2,301)

Manual transmission (lbs./kg)	5,050 (2,291)

*weights differ by transmission called out

### Accommodations

Seating Capacity (front/rear)	2/3

SAE Total Interior Passenger Volume, 
(cu. ft. / cu. m)	103 (2.91)

Front

Headroom	42.8 (1,087) soft-top / 40.8 (1036) hardtop

Legroom	41.2 (1,038)

Shoulder room	55.7 (1,417)

Hip room	53.9 (1,370)

Seat travel	9.4 (239.5)

EPA front volume index	53.7 (1.52)

Rear

Headroom	42.8 (1,087) soft-top / 40.8 (1036) hardtop

Legroom	38.3 (966)

Knee clearance	2.7 (69)

Shoulder room	55.7 (1,416)

Hip room	55.9 (1,421)

EPA rear seat volume index 
(cu. ft. / cu. m) 	49.2 (1.39)

Liftover Height	29.7 (755)

### WHEELS

Availability	Standard (Sport)

Type and material	Fully painted steel with Low Gloss Black

Size	17 x 7.5 in.

Availability	Standard (Sport S)

Type and material	Fully painted cast aluminum with High Gloss Tech Silver

Size	17 x 7.5 in.

Availability	Optional (Sport S)

Type and material	Fully painted cast aluminum with High Gloss Black

Size	17 x 7.5 in.

Availability 	Standard (Overland)

Type and material	Fully painted cast aluminum with High Gloss Granite Crystal

Size	18 x 7.5 in.

Availability	Standard (Mojave)

Type and material	Fully painted cast aluminum with Low Gloss Global Black

Size	17 x 7.5 in.

### WHEELS (CONTINUED)

Availability	Optional (Mojave)

Type and material	Fully painted cast aluminum with polish / painted textured Low Gloss Black

Size	17 x 7.5 in.

Availability	Standard (Rubicon)

Type and material	Fully painted cast aluminum with Granite Crystal

Size	17 x 7.5 in.

Availability	Optional (Rubicon)

Type and material	Painted pocket with Mid-Gloss paint and polished cast aluminum

Size	17 x 7.5 in.

### TIRES

Availability	Standard (Sport)

Size and type	245/75R17, on/off-road, black sidewall

Mfr. and model	Bridgestone Dueler H/T 685, Bridgestone Dueler A/T RH-S

Revs per mile (km)	661 (411) Bridgestone Dueler H/T 685 and A/T RH-S

Availability	Standard (Sport S)

Size and type	245/75R17, on/off-road, black sidewall

Mfr. and model	Bridgestone Dueler H/T 685, Bridgestone Dueler A/T RH-S

Revs per mile (km)	661 (411)

Availability                                                                     Standard (Willys)

Size and type                                                            LT255/75R17

Mfr. and model                                                          Firestone Destination M/T2

Revs per mile (km)                                                    650 (404)

Availability                                                                     Optional (Willys)

Size and type                                                            LT255/75R17

Mfr. and model                                                          BFGoodrich Mud-Terrain T/A KM2

Revs per mile (km)                                                    649 (403)

Availability	Standard (Overland)

Size and type	255/70R18

Mfr. and model	Bridgestone Dueler H/T 685

Revs per mile (km)	650 (404)

Availability	Optional (Overland)

Size and type	255/70R18

Mfr. and model	Bridgestone Dueler A/T RH-S

Revs per mile (km)	650 (404)

### TIRES (CONTINUED)

Availability	Standard (Mojave)

Size and type	LT285/70R17C

Mfr. and model	Falken Wildpeak A/T

Revs per mile (km)	617 (383)

Availability	Optional (Mojave)

Size and type	LT285/70R17C

Mfr. and model	Falken Wildpeak M/T

Revs per mile (km)	617 (383)

Availability	Standard (Rubicon)

Size and type	LT285/70R17C

Mfr. and model	Falken Wildpeak A/T

Revs per mile (km)	617 (383)

Availability	Optional (Rubicon)

Size and type	LT285/70R17C

Mfr. and model	Falken Wildpeak M/T

Revs per mile (km)	617 (383)

Four–wheel Antilock Brakes	Standard

Electronic Stability Control	Standard

All-speed Traction Control	Standard

### Towing Capacities(1)

#### GLADIATOR SPORT

| Engine | Trans Type | Transmission | Axle Ratio | Payload | Base Wt. | Base Wt. Front | Base Wt. Rear | GAWR Front | GAWR Rear | GCWR | Max Trailer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 3.73 | 1,710 | 4,650 | 2,465 | 2,186 | 3,100 | 3,750 | 9,100 | 4,000 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 3.73 | 1,105 | 4,691 | 2,486 | 2,205 | 3,100 | 3,750 | 9,650 | 4,500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 4.10 | 1,535 | 4,712 | 2,497 | 2,215 | 3,100 | 3,750 | 12,800 | 7,700 |
| 3.0-liter diesel | A8 | 8HP75 8-speed automatic | 3.73 | 1,325 | 4,988 | 2,733 | 2,255 | 3,200 | 3,750 | 11,800 | 6,500 |
| 3.0-liter diesel* | A8 | 8HP75 8-speed automatic | 3.73 | 1,325 | 4,972 | 2,725 | 2,247 | 3,200 | 3,750 | 11,800 | 6,500 |

1. GCWR = Base Weight + Maximum Trailer Weight + 150 lbs. for driver.

* Sport S model

#### GLADIATOR OVERLAND

| Engine | Trans Type | Transmission | Axle Ratio | Payload | Base Wt. | Base Wt. Front | Base Wt. Rear | GAWR Front | GAWR Rear | GCWR | Max Trailer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 3.73 | 1,140 | 4,720 | 2,502 | 2,218 | 3,100 | 3,750 | 9,100 | 4,000 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 3.73 | 1,120 | 4,676 | 2,478 | 2,198 | 3,100 | 3,750 | 11,100 | 6,000 |
| 3.0-liter diesel | A8 | 8HP75 8-speed automatic | 3.73 | 1,275 | 5,041 | 2,756 | 2,285 | 3,200 | 3,750 | 11,800 | 6,500 |

1. GCWR = Base Weight + Maximum Trailer Weight + 150 lbs. for driver.

#### GLADIATOR MOJAVE

| Engine | Trans Type | Transmission | Axle Ratio | Payload | Base Wt. | Base Wt. Front | Base Wt. Rear | GAWR Front | GAWR Rear | GCWR | Max Trailer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 4.10 | 1,200 | 4,948 | 2,622 | 2,326 | 3,100 | 3,750 | 9,900 | 4,500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 4.10 | 1,200 | 4,970 | 2,634 | 2,336 | 3,100 | 3,750 | 11,450 | 6,000 |

1. GCWR = Base Weight + Maximum Trailer Weight + 150 lbs. for driver.

#### GLADIATOR RUBICON

| Engine | Trans Type | Transmission | Axle Ratio | Payload | Base Wt. | Base Wt. Front | Base Wt. Rear | GAWR Front | GAWR Rear | GCWR | Max Trailer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 4.10 | 1,200 | 5,050 | 2,677 | 2,374 | 3,100 | 3,750 | 10,000 | 4,500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 4.10 | 1,160 | 5,072 | 2,688 | 2,384 | 3,100 | 3,750 | 12,450 | 7,000 |
| 3.0-liter diesel | A8 | 8HP75 8-speed automatic | 3.73 | 1,075 | 5,352 | 2,887 | 2,465 | 3,400 | 3,750 | 12,100 | 6,000 |

1. GCWR = Base Weight + Maximum Trailer Weight + 150 lbs. for driver.

###