# 2023 Jeep® Grand Cherokee 4xe

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Vehicle Type	Four-door sport-utility vehicle

Assembly Plant	Detroit Assembly Complex – Mack Plant, Detroit, Michigan

EPA Vehicle Class	Multipurpose vehicle

Introduction Date	Summer 2022 as a 2022 model

### BODY AND CHASSIS

Layout	Front engine, four-wheel drive

Construction	Steel uniframe

### TRANSFER CASE: MP3028C

Availability	Standard

Type	Two-speed electronically shifted

Operating Modes	4x4 Low (Lock), Neutral; Full-time Active 4x4

Low Range Ratio	2.72

Torque Split, Front/Rear	Variable

### ENGINE: 2.0-LITER TURBO I-4

Availability	Standard

Displacement	121 cu. in. (1,995 cu. cm)

Bore x Stroke	3.31 x 3.54 (84 x 90)

Valve System	Chain-driven DOHC, 16 valves

Fuel Injection	Direct Injection

Construction	Aluminum block, aluminum alloy heads

Compression Ratio	10.0:1

Power (SAE J2723)	270 hp (200kW) @ 5,250 rpm

Torque (SAE J2723)	295 lb.-ft. (400 N•m) @ 3,000 rpm

Max. Engine Speed	5,800 rpm (electronically limited)

### ENGINE: 2.0-LITER TURBO I-4 (CONTINUED)

Fuel Requirement	Minimum unleaded regular, 87 octane (R + M)/2; 91 octane or higher recommended for optimum fuel economy and performance

Fuel Tank Capacity (gallons)	19.0

Oil Capacity	5.0 quarts (4.7 liters)

Coolant Capacity	9.9 quarts (9.4 liters)

Emission Controls 	GPEC5 engine-management system with close-coupled catalyst and wide range O2 sensor

Max. Gross Trailer Weight	6,000 lbs.

Assembly Plant	Termoli, Italy

### eTORQUE BELT-START GENERATOR/MOTOR

Availability	Standard

Peak power	44 hp (33 kW)

Starting torque	39 lb.-ft. (53 Nm)

### TRANSMISSION: 8P75PH PHEV AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard

Description	Plug-in hybrid automatic transmission with AC traction motor with Motor Interrupting Clutch and Integrated Launch Element; adaptive electronic control and paddle shifter driver-interactive control

Gear Ratios

1st	4.71

2nd 	3.14

3rd 	2.11

4th 	1.67

5th 	1.28

6th 	1.00

7th 	0.84

8th 	0.67

Reverse 	3.32

### INTEGRATED TRANSMISSION TRACTION MOTOR

Availability	Standard

Peak torque	195 lb.-ft. (265 Nm)

Peak power	134 hp (100 kW)

### HIGH VOLTAGE BATTERY

Availability	Standard

Chemistry	Lithium Ion Li-NMC

Charging Modes	Level 1 — 120V AC, Level 2 — 240V AC

Operating Voltage	260 – 400 VDC

Gross Capacity	17.3 kWh

Maximum Charge Rate	7.2 kW

### TOTAL POWERTRAIN OUTPUT

Availability	Standard

Peak combined torque	470 lb.-ft. (637 Nm) at 3,000 rpm

Peak combined power	375 hp (280 kW) at 5,250 rpm

### AXLES

Front

Differential type	Conventional with Front Axle Disconnect

Availability	Standard

Ring gear diameter	7.7 in. (195mm)

Axle ratio	3.70:1

Rear

Differential type	Conventional (standard Electronic Limited Slip on Trailhawk and Summit, optional on Overland)

Availability	Standard on all engines

Ring gear diameter 	9.1 in. (230mm)

Axle ratio	3.70:1

### ELECTRICAL SYSTEM

Battery	Maintenance-free

H7 700 CCA + Auxiliary 200 CCA on engine stop-start (ESS)

### SUSPENSION

Front	Multi-link independent front suspension, coil springs, gas-charged, 
twin-tube coilover shock absorbers, all aluminum arms and knuckles

Rear	Multi-link rear suspension, coil spring, twin tube shocks (including load leveling for towing), all aluminum arms and knuckles (including extruded aluminum spring links)

### STEERING

Type	Electric rack-and-pinion steering

Overall Ratio	15.6:1

Turning Diameter (curb-to-curb) 	38 ft. (11.58m)

Steering Turns (lock-to-lock)	3.0

### BRAKES

Type	Electric boost

Availability	Standard

Front

Rotor size and type Inches (mm)	13.94 x 1.10 (354 x 28) vented disc

Caliper size and type Inches (mm)	2.00 (51) two-piston pin-slider caliper

Swept area, (sq. in. / sq. cm)		377 (2,432.25)

Rear

Rotor size and type Inches (mm)	13.78 x 0.87 (350 x 22) vented disc

Caliper size and type Inches (mm)	2.00 (51) single-piston pin-slider EPB caliper

Swept area, (sq. in. / sq. cm)	293.28 (1,892.12)

### DIMENSIONS AND CAPACITIES

Wheelbase	116.7 (2,964)

Track, Front	65.4 (1,660)

Track, Rear	65.4 (1,660)

Overall Length	193.5 (4914)

Overall Width (width at mirrors)	84.6 (2,149)

Overall Width (width with folded mirrors)	77.5 (1,968)

Body Width	77.5 (1,968)

Overall Height	70.8 (1,799) — standard suspension

70.9 (1,801) — air suspension

Load Floor Height	32.4 (823) — standard suspension

30.4 (772) — air suspension (with entry/exit enabled)

Sill Step Height	20.2 (512) — standard suspension

18.4 (467) — air suspension (with entry/exit enabled)

Ground Clearance 	8.4 (214) — standard suspension

10.9 (276) — air suspension (Pos#2)

10.9 (278) — air suspension (Trailhawk Pos #2)

### DIMENSIONS AND CAPACITIES (continued)

Chassis (fuel tank) 	10.3 (261) — standard suspension

12.6 (320) — air suspension (Pos#2)

13.2 (335) — air suspension (Trailhawk Pos#2)

Front Axle 	8.4 (214) — standard suspension

10.9 (277) — air suspension (Pos#2)

11.3 (287) — air suspension (Trailhawk Pos#2)

Rear Axle 	9.1 (230) — standard suspension

11.7 (297) — air suspension (Pos#2)

12.5 (317) — air suspension (Trailhawk Pos#2)

Approach Angle (degrees)	20.5 — standard suspension

30.1 — air suspension (Overland, Pos#2)

28.2 — air suspension (Summit, Pos#2)

35.7 — air suspension (Trailhawk Pos#2)

Ramp Breakover Angle (degrees)	16.3 — standard suspension

20.9 — air suspension (Pos#2)

22.3 — air suspension (Trailhawk Pos#2)

Departure Angle (degrees)	26.6 — standard suspension

30.0 — air suspension (Trailhawk Pos#2)

Frontal Area sq. ft. 	31.7

Drag Coefficient (Cd)	0.357

Aero CdA	11.3 (Cd x frontal area)

Fuel Tank Capacity, (gallons / liter)	19 (72)

### ACCOMMODATIONS

Seating Capacity (front / rear) 	2/3 (second-row bench)

SAE Total Interior Passenger Volume, 
(cu. ft. / cu. m)	144.6 (4.1)

Front Row

Headroom	39.9 (1,013)

Legroom	41.3 (1,050)

Shoulder room	59.2 (1,504)

Hip room	57.4 (1,459)

Seat travel	9.8 (250)

EPA front volume index (cu. ft. / cu. m)	56.5 (1.6)

### ACCOMMODATIONS (CONTINUED)

Second Row

Headroom	39.4 (1,002)

Legroom	38.2 (971)

Knee clearance	4.1 (103)

Shoulder room	58.0 (1,474)

Hip room	56.3 (1,429)

EPA rear seat volume index (cu. ft. / cu. m)	50.5 (1.4)

Cargo volume behind first-row seats (cu. ft. / cu. m)	70.8 (2.0)

Behind second-row seats (cu. ft. / cu. m)	37.7 (1.07)

### WEIGHTS (Estimates)

| WEIGHTS (Estimates) | MODEL |  | GVWR(a) lbs. (kg) | CURB WEIGHT(b) lbs. (kg) | PAYLOAD(c) lbs. (kg) |
| --- | --- | --- | --- | --- | --- |
|  | 4xe |  | 6,900 (3,130) | 5,368 (2,435) | 1,050 (476) |
|  | Trailhawk 4xe |  | 6,900 (3,130) | 5,521 (2,504) | 1,050 (476) |
|  | Overland 4xe |  | 6,900 (3,130) | 5,484 (2,487) | 1,050 (476) |
|  | Summit 4xe |  | 6,900 (3,130) | 5,507 (2,498) | 1,050 (476) |
| (a) Gross Vehicle Weight Rating (b) Curb weight includes standard equipment and full quantities of fuel, lubricant and coolant. (c) Payload is the maximum allowable weight of driver, passengers, cargo and options, rounded to the nearest 10 lbs. (5kg). |  |  |  |  |  |

### WEIGHT DISTRIBUTION, F/R

| WEIGHT DISTRIBUTION, F/R | MODEL |  | DISTRIBUTION |
| --- | --- | --- | --- |
|  | 4xe |  | 53/47 |
|  | Trailhawk 4xe |  | 52/48 |
|  | Overland 4xe |  | 52/48 |
|  | Summit 4xe |  | 52/48 |

### Wheels

Availability	Standard — 4xe

Type and material	Fully painted aluminum

Size (inches)	18 x 8

### Wheels (continued)

Availability	Optional — 4xe

Type and material	Polished / painted aluminum

Size (inches)	20 x 8.5

Availability	Standard — Trailhawk 4xe

Type and material	Machined face with Mid-Gloss Granite Crystal with Blue Accent

Size (inches)	18 x 8

Availability	Optional — Trailhawk 4xe (late availability)

Type and material	Black-painted with Blue Accent

Size (inches)	18 x 8

Availability	Standard — Overland 4xe

Type and material	Machined cast aluminum with Black Noise pockets

Size (inches)	20 x 8.5

Availability	Optional — Overland 4xe (packaged with Off-road Group)

Type and material	Polished aluminum with High-Gloss Black pockets

Size (inches)	18 x 8

Availability	Standard — Summit 4xe

Type and material	Polished cast aluminum Painted Silver Lithos

Size (inches)	20 x 8.5

Availability 	Standard — Summit Reserve 4xe

Type and material	Machined cast aluminum with Mid-Gloss Black pockets

Size (inches)	21 x 9

Availability 	Standard — Summit High Altitude 4xe, Summit Reserve High Altitude 4xe

Type and material	Black-painted cast aluminum

Size (inches)	21 x 9

### TIRES

Availability	Standard — 4xe

Size and type	265/60R18

Mfr. and model	Michelin Primacy XC All-Season or Michelin Primacy LTX All-Season

Revs per kilometer	422

Availability 	Standard — Trailhawk 4xe

Size and type	265/60R18

Mfr. and model	Goodyear Wrangler Territory AT

Revs per kilometer	425

Availability 	Optional — Overland 4xe (packaged with Off-road Group)

Size and type	265/60R18

### TIRES (continued)

Mfr. and model	Goodyear Wrangler All-Terrain Adventurer

Revs per kilometer	422

Availability	Standard — Summit 4xe

Size and type	265/50R20

Mfr. and model	Pirelli Scorpion Verde Black Sidewall All-Season

Revs per kilometer	419

Availability	Optional — Summit 4xe (included with Summit Reserve and High Altitude Packages)

Size and type	275/45R21

Mfr. and model	Continental CrossContact

Revs per kilometer	419

Parking Brake Type	Electric

Four-wheel Antilock Brakes	Standard

Electronic Stability Control	Standard

All-speed Traction Control	Standard

Brake Assist	Standard

### TRAILER TOWING

|  |  | AXLE RATIO | MAXIMUM TRAILER WEIGHT LBS. (KG) |
| --- | --- | --- | --- |
| 4WD |  | 3.70 | 6,000 (2,722) |

###