# 2023 Jeep® Renegade

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Body Style 	Sport-utility vehicle (SUV)

Assembly Plant	Melfi, Italy

EPA Vehicle Class 	Multipurpose vehicle

### BODY AND CHASSIS

Layout	Transverse front engine, 4x4

Construction 	Steel uniframe

### ENGINE: 1.3-LITER TURBOCHARGED

Availability	Standard

Type and Description	Inline four-cylinder, liquid-cooled, turbocharged

Displacement	81 cu. in. (1,332 cu. cm.)

Bore x Stroke	2.76 x 3.4 (70 x 86.5)

Valve System	SOHC & hydraulic valve actuation

Fuel Injection	Direct injection

Construction	Aluminum block and aluminum heads

Compression Ratio	10.5

Maximum Turbo Boost (psi / bar)	2.7 bar

Power (SAE)	177 hp (132 kW) @ 5,750 rpm

Torque (SAE)	210 lb.-ft. (270 N•m) @ 1,750 rpm

Max. Engine Speed	6,500 rpm (electronically limited)

Fuel Requirement	87 octane (R+M)/2 acceptable

91 octane recommended

Oil Capacity	4.5 quarts (4.3 liters) with dry filter

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors and internal engine features

Fuel Economy (city/hwy/combined)	4x4: 23/29/26

Towing	2,000 lbs. (907 kg) (not recommended for FWD)

Engine Assembly Plant	Bielsko-Biała, Poland

### TRANSMISSION: 948TE NINE-SPEED AUTOMATIC

Availability	1.3-liter Turbo engine (4x4)

Description 	Planetary gear train, transverse layout

Ratio Spread	9.81

Gear Ratios

1st	4.71

2nd	2.84

3rd	1.91

4th	1.38

5th	1.00

6th	0.81

7th	0.70

8th	0.58

9th	0.48

Reverse	3.83

Final Drive Ratio	3.734

4.585 (Trailhawk)

### SUSPENSION

Front 	MacPherson strut, coil springs, flat front steel crossmember, 
high-strength steel double shell lower control arms for 4x4, stabilizer bar

Rear	Chapman strut, high-strength steel links, isolated steel rear cradle for 4x4, stabilizer bar

### STEERING

Type 	Electric power rack and pinion

Overall Ratio 	15.7

Turning Diameter (curb-to-curb) 	36.3 (11.07) 4x4

35.3 (10.76) Trailhawk 4x4

Steering Turns (lock-to-lock)	2.68 (4x4)

2.76 (Trailhawk 4x4)

### BRAKES

Front

Size and type	12 x 1.1 (305 x 28) vented rotor with 2.36 (60) single-piston 
floating caliper

Swept area (per caliper)	138.77 sq. in. (895.3 sq. cm), 1 x 60

115.88 sq. in. (747.6 sq. cm), 1 x 57

Rear

Size and type 	10.95 x 0.47 (278 x 12) solid rotor 
with 1.5 (38) single-piston floating caliper

Swept area (per caliper)	101.7 sq. in. (656.4 sq. cm), 1 x 38

Power-assist Type 	10 (254) vacuum assist

Four-wheel Antilock Brake System (ABS)	Standard

Electronic Stability Control (ESC)	Standard

Parking Brake Type	Electric motor on caliper

### DIMENSIONS AND CAPACITIES

Wheelbase	101.2 (2,570)

Track, Front	60.6 (1,540)

Track, Rear	60.6 (1,540)

Overall Length	166.6 (4,232)

Overall Width (with folded side mirrors)	73.9 (1,886.8)

Overall Width (with open side mirrors)	79.6 (2,023.1)

Overall Height 	66.5 (1,689)

Load-floor Height	29.8 (758)

Sill-step Height	19.4 (494) front, 19.6 (497) rear

Ground Clearances 	8.0 (203) 4x4 (8.0 – 203)

8.7 (220) Trailhawk 4x4

Approach Angle (with air dam) (degrees)	21.0 4x4

30.5 Trailhawk 4x4

Breakover Angle (degrees)	24.0 4x4

25.7 Trailhawk 4x4

Departure Angle (degrees)	32.1 4x4

34.3 Trailhawk 4x4

### DIMENSIONS AND CAPACITIES (CONTINUED)

Curb Weight (kg)	3,320 (1,513) 4x4 and 1.3-liter Turbocharged

3,527 (1,602) Trailhawk 4x4 and 1.3-liter Turbocharged

Aero Cd	0.36 4x4

0.37 Trailhawk 4x4

Fuel Tank Capacity	12.7 gal. (48 liters)

### ACCOMMODATIONS

Seating Capacity — Front/Rear 	2/3

Front Row

Headroom without sunroof	41.1 (1,045)

Headroom with sunroof	39.7 (1,009)

Legroom 	41.2 (1,046)

Shoulder room 	55.9 (1,419)

Hip room	53.1 (1,348)

Seat travel 	9.0 (230) driver / 10.2 (260) front passenger

Recliner range (degrees)	From 25 nominal, 38 forward and 30 rearward 
(from vertical 13 forward, 55 rearward)

SAE volume (standard roof), cu. ft. (cu. m)	54.8 (1.55)

Second Row

Headroom 	40.5 (1,029)

Legroom 	35.1 (891)

Shoulder room 	55.1 (1,400)

Hip room	51.9 (1,319)

SAE volume, cu. ft. (cu. m)	45.3 (1.28)

Cargo Access

Liftover height	30.2 (766)

Maximum cargo width at liftgate opening	40.1 (1,020)

Minimum cargo width at liftgate opening 	39.0 (990) at bottom of opening

Maximum cargo height at liftgate opening	29.5 (749)

Minimum cargo height at liftgate opening	29.0 (739)

Distance between wheelhouse interior trim	37.6 (956)

### ACCOMMODATIONS (CONTINUED)

SAE Cargo Volume

Rear seats up, cu. ft. (cu. m)	18.5 (0.525)

Rear seats folded, cu. ft. (cu. m)	50.8 (1.440)

EPA interior volume index, cu. ft. (cu. m)	118.6 (3.359)

Behind rear seat, cu. ft. (cu. m)	18.5 (0.525) (rear seat at design position)

Behind front-row seats 
with rear seats folded, cu. ft. (cu. m)	50.8 (1.440)

### TIRES

Availability	Standard — Latitude

Size and type 	215/60R17 BSW All-Season

Mfr. and model 	Continental, CrossContact LX Sport

Revs per mile (km)	768 (476)

Availability	Standard — Trailhawk and Upland Edition

Size and type	215/65R17 OWL All-Season

Mfr. and model	Falken WildPeak H/T

Revs per mile (km)	722 (448)

Availability	Optional — High Altitude

Size and type	235/45R19 BSW All-Season

Mfr. and model	Goodyear Eagle LS-2 Runonflat

Revs per mile (km)	760 (471)

Availability	Standard — Limited and Altitude

Size and type	225/55R18 BSW All-Season

Mfr. and model	Goodyear Eagle Sport AS

###