# 2023 Jeep® Wrangler

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

Jeep Wrangler 4xe features and specs are included in 4xe specific documents. 
Please reference Wrangler 4xe tab on the media site.

### General Information

Vehicle Type	Sport-utility vehicle

Assembly Plant	Toledo Supplier Park, Toledo, Ohio

EPA Vehicle Class	Multipurpose vehicle

Introduction Date	Fall 2017 as a 2018 model

### BODY/CHASSIS

Layout	Longitudinal front engine, four-wheel drive

Construction	Ladder-type frame, open steel and aluminum body

### Engine: 3.6-LITER PENTASTAR V-6

Availability	Standard (all engine stop-start, MTX only) — Sport, Rubicon

Available (all eTorque mild-hybrid assist, ATX only) — Sport, Sahara, Rubicon

Type and Description	60-degree, V-type, liquid-cooled

Displacement (cu. in. / cc)	220 cu. in. (3,604 cu. cm)

Bore x Stroke 	3.78 x 3.27 (96 x 83)

Valve System	Chain-driven DOHC, 24 valves and hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block, aluminum alloy heads

Compression Ratio	11.3:1

Power (SAE net) 
(hp / kW@rpm)	285 hp (213 kW) at 6,400 rpm

Torque (SAE net) 
(lb.-ft. / N•m)	260 lb.-ft. (353 N•m) at 4,800 rpm

Max. Engine Speed (rpm)	6,600 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane (R + M)/2

Fuel Tank Capacity (gallons)	17.5 (2-door), 21.5 (4-door)

### Engine: 3.6-LITER PENTASTAR V-6 (CONTINUED)

Oil Capacity 	5.0 quarts (4.7 liters)

Coolant Capacity 	10 quarts (10.1 liters)

Emission Controls 	Dual three-way catalytic converters, heated oxygen sensors and internal engine features

Max. Gross Trailer Weight	2,000 lbs. (2-door), 3,500 lbs. (4-door)

EPA Fuel Economy mpg (city/hwy/combined)	17/25/20 — manual with engine stop-start (ESS) (2-door)

17/23/19 — manual with ESS (4-door)

18/23/20 — automatic with eTorque mild-hybrid assist (2-door)

19/22/20 — automatic with eTorque mild-hybrid assist (4-door)

Assembly Plant 	Saltillo South Engine Plant, Saltillo, Coahuila, Mexico

### Engine: 2.0-LITER TURBO I-4

Availability	Standard — Sahara

Available — Sport, Rubicon (all ESS)

Type and Description	I-4 16-valve with direct injection, turbo charging, with throttled, cooled exhaust gas recirculation (EGR)

Displacement (cu. in. / cc)	121 cu. in. (1995 cu. cm)

Bore x Stroke 	3.31 x 3.54 (84 x 90)

Valve System	Chain-driven DOHC, 16 valves

Fuel Injection	Direct injection

Construction	Aluminum block, aluminum alloy heads

Compression Ratio	10.0:1

Power (SAE net) (hp / kW @ rpm)	270 hp (200 kW) at 5,250 rpm

Torque (SAE net) (lb.-ft. / N•m)	295 lb.-ft. (400 N•m) at 3,000 rpm

Max. Engine Speed (rpm)	5,800 rpm (electronically limited)

Fuel Requirement	Minimum unleaded regular, 87 octane (R + M)/2; 91 octane or higher 
recommended for optimum fuel economy and performance

Fuel Tank Capacity (gallons)	17.5 (2-door), 21.5 (4-door)

Oil Capacity 	5 quarts (4.7 liters)

Coolant Capacity 	9.9 quarts (9.4 liters)

Emission Controls 	GPEC4 engine-management system with close-coupled catalyst 
and wide range O2 sensor

Max. Gross Trailer Weight	2,000 lbs. (2-door), 3,500 lbs. (4-door)

EPA Fuel Economy mpg (city/hwy/combined)	22/24/23 — automatic with ESS (2-door)

21/22/21 — automatic with ESS (4-door)

Assembly Plant	Termoli, Italy and Trenton South Engine Plant, Trenton, Michigan

### ENGINE: 3.0-LITER ECODIESEL V-6

Availability	Available — Sport, Rubicon, Sahara (all ESS, 4-door only)

Type and Description	60-degree diesel V-6

Displacement	182 cu. in. (2,987 cu. cm)

Bore x Stroke	3.27 x 3.60 (83 x 92)

Valve System	Chain driven DOHC, 24 valves, hydraulic lash adjusters with roller fingers follower

Fuel Injection	Common rail, 29,000 psi (2000 bar), solenoid injectors

Construction	Compacted graphite iron cylinder block, aluminum alloy heads

Compression Ratio	16.0:1

Power 	260 hp (194 kW) @ 3,600 rpm

Torque 	442 lb.-ft. (600 N•m) @ 1,400-2,800 rpm

Max. Engine Speed	4,600 rpm

Fuel Requirement	Ultra-low sulfur diesel

Fuel Tank Capacity (gallons)	18.3 (4-door)

Oil Capacity	9.0 quarts (8.5 liters)

Coolant Capacity	12.3 quarts (11.6 liters)

Emission Controls	High-pressure EGR, low-pressure EGR, oxidation catalyst, diesel particulate filter, SCR with urea injection

Max. Gross Trailer Weight	3,500 lbs. (4-door)

EPA Fuel Economy mpg (city/hwy)	22/29/25 — automatic with ESS (4-door only)

Assembly Plant	Cento, Italy

### ENGINE: 6.4-LITER V-8

Availability	Standard — Rubicon 392

Type and Description	90-degree V-type, liquid cooled

Displacement (cu. in. / cc)	392 cu. in. (6,417 cu. cm)

Bore x Stroke	4.09 x 3.72 (103.9 x 94.6)

Valve System	Pushrod-operated overhead valves, 16 valves with sodium-filled exhaust valves and hollow stem intake valves, 16 conventional hydraulic lifters, all with roller tips

Fuel Injection	Sequential, multi-port, electronic, returnless; Fuel Saver mode

Construction	Deep-skirt cast-iron block with cross-bolted main bearing caps, aluminum alloy heads with hemispherical combustion chambers

Compression Ratio 	10.9:1

Power (SAE net) (hp / kW @ rpm)	470 hp (354 kW) @ 6,000 rpm

Torque (SAE net) (lb.-ft. / N•m)	470 lb.-ft. (637 N•m) @ 4,300 rpm

Max. Engine Speed (rpm)	6,400 rpm (electronically limited)

### ENGINE: 6.4-LITER V-8 (CONTINUED)

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Fuel Tank Capacity (gallons)	21.5

Oil Capacity	7 quarts (6.6 liters)

Coolant Capacity	14 quarts (13.25 liters)

Emission Controls	Dual close-coupled three-way catalytic converters, 
quad-heated oxygen sensors and internal engine features

Max. Gross Trailer Weight	3,500 lbs.

Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### Transmission: 850RE automatic, Eight-speed overdrive

Availability	Standard — 2.0-liter I-4 ESS

Optional — 3.6-liter V-6 with eTorque mild-hybrid assist

Description	Adaptive electronic control or Electronic Range Select (ERS) driver-interactive manual control and electronically modulated torque converter clutch

Gear Ratios

1st	4.71

2nd	3.13

3rd	2.10

4th	1.67

5th	1.28

6th	1.00

7th	0.84

8th	0.67

Reverse	3.30

### Transmission: 8HP75 AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard — 3.0-liter EcoDiesel V-6 ESS, 6.4-liter V-8

Description	Adaptive electronic control or ERS driver-interactive manual control and electronically modulated torque converter clutch

Gear Ratios

1st	4.71

2nd	3.13

3rd	2.10

4th	1.67

5th	1.28

6th	1.00

### Transmission: 8HP75 AUTOMATIC, EIGHT-SPEED OVERDRIVE (continued)

7th	0.84

8th	0.67

Reverse	3.30

### TRANSMISSION: D478 MANUAL, SIX-SPEED WITH OVERDRIVE

Availability	Standard — 3.6-liter V-6 with engine stop-start

Description	Synchronized in all forward gears and Reverse, multi-rail shift system with top-mounted shift lever

Gear Ratios

1st	5.13

2nd	2.63

3rd	1.53

4th	1.00

5th	0.81

6th	0.72

Reverse	4.49

Final Drive Ratio	3.45 standard, 3.73 optional (4.10 Rubicon)

### TRANSFER CASE: NV241 command-trac

Available	Standard — Sport and Sahara

Type	Part-time

Operating Modes	2WD High; 4WD High; Neutral; 4WD Low

Low Range Ratio	2.72:1

### TRANSFER CASE: MP3022 Selec-Trac

Available 	Standard — Rubicon 392, Optional — Sport and Sahara

Type	Full-time

Operating Modes	2WD High (N/A 392); 4WD Auto; 4WD High; Neutral; 4WD Low

Low Range Ratio	2.72:1

### TRANSFER CASE: MP3022OR ROCK-TRAC

Available 	Optional — Rubicon

Type	Full-time

Operating Modes	2WD High; 4WD Auto; 4WD High; Neutral; 4WD Low

Low Range Ratio	4.0:1

### TRANSFER CASE: NV241OR Rock-trac

Available	Standard — Rubicon

Type	Standard — Part-time

Operating Modes	2WD High; 4WD High; Neutral; 4WD Low

Low Range Ratio	4.0:1

### Axles

Front	3rd generation Dana axles

Differential type	Open or Tru-Lok electronic locking (Rubicon models)

Rear	3rd generation Dana axles

Differential type	Open (Sport, Sahara) with available Trac-Lok anti-spin, 
Tru-Lok electronic locking (Rubicon models)

Axle ratios 	3.45, 3.73 (EcoDiesel, Rubicon 392), 4.10 (Rubicon), 
4.56 (opt Rubicon and Rubicon 392), 4.88 (opt Rubicon 3.6 liters)

### Electrical System

Architecture	Powernet

Alternator (standard)	180 amp

Battery (standard)	650 CCA, maintenance free

### ELECTRICAL SYSTEM (Rubicon 392)

Architecture	Powernet

Alternator	220 amp

Battery	700 CCA, maintenance free

### Suspension

Front	Solid axle, link coil, leading arms, track bar, coil springs, stabilizer bar

Stabilizer bar	Electronic sway-bar disconnect system — (Rubicon models)

Shock type	Sport: Gas-charged twin-tube shock absorbers with full displacement 
Multi-tuned Valve (MTV) technology

Sahara: High-pressure gas-charged monotube shock absorbers with MTV technology

Rubicon: High-pressure gas-charged monotube shock absorbers with MTV technology and hydraulic rebound stop

Rubicon 392: High-pressure gas-charged FOX aluminum monotube shock absorbers with hydraulic rebound stop

Rubicon 392 Xtreme Recon: High-pressure gas-charged monotube shock absorbers with MTV technology and hydraulic rebound stop

Rear	Solid axle, link coil, trailing arms, track bar, coil springs, stabilizer bar

Shock type	Sport: Gas-charged twin-tube shock absorbers with full displacement MTV technology

Sahara: High-pressure gas-charged monotube shock absorbers with MTV technology

Rubicon: High-pressure gas-charged monotube shock absorbers with MTV technology and hydraulic rebound stop

Rubicon 392: High-pressure gas-charged FOX aluminum monotube shock absorbers with hydraulic rebound stop

Rubicon 392 Xtreme Recon: High-pressure gas-charged monotube shock absorbers with MTV technology and hydraulic rebound stop

### STEERING

Type	Electro-hydraulic power

Overall ratio	17.7:1 (2-door), 15.6:1 (4-door), 15.1:1 (EcoDiesel, Xtreme Recon Pkg), 13.9 (Rubicon 392), 14.5 (Rubicon 392 Xtreme Recon)

Turning diameter (curb-to-curb) (ft./m)	32.7 / 10.0 (2-door), 38.8 / 11.8 (4-door)

Steering Turns (lock-to-lock)	3.6 (2-door), 3.2 (4-door), 3.1 (EcoDiesel, Rubicon 392, Xtreme Recon Pkg)

P/S Pump	Electro-hydraulic

### BRAKES

Type	Power-assisted, anti-lock brake system

Front

Rotor size and type	12.9 x .94 (330 x 24) vented rotor — Sport

12.9 x 1.1 (330 x 28) vented rotor — Sahara and Rubicon models

Caliper size and type	1.88 (48) twin-piston floating caliper — Sport

2 (51) twin-piston floating caliper — Sahara, Rubicon

Rear

Rotor size and type	12.9 x .47 (328 x 12) solid rotor — Sport

13.4 x .55 (342 x 14) solid rotor — Sahara and Rubicon

13.6 x .86 (345 x 22) vented rotor — Xtreme Recon Pkg

14 x .86 (350 x 22) vented rotor — Rubicon 392

Caliper size and type	1.77 (45) single-piston floating caliper — Sport

1.88 (48) single-piston floating caliper — Sahara and Rubicon

2 (51) single-piston floating calipers — Rubicon 392, Xtreme Recon Pkg

### Dimensions and Capacities

Wheelbase	96.8 (2,460) — 2-door, 118.4 (3,008) — 4-door

Overhang — Front 	29.2 (741.6)

29.5 (749.8) — Rubicon 392

Overhang — Rear	40.8 (1,036.3)

40.3 (1,023) — Rubicon 392

Track — Front	62.9 (1,598)

64.4 (1,636) — Rubicon, Rubicon 392

66.9 (1,699) — Xtreme Recon Pkg

Track — Rear	62.9 (1,598)

64.4 (1,636) — Rubicon, Rubicon 392

66.9 (1,699) — Xtreme Recon Pkg

Overall Length (includes spare tire)	166.8 (4,237) — 2-door

188.4 (4,785) — 4-door

192.5 (4,890) — Xtreme Recon Pkg

Overall Width	73.9 (1,877)

79.3 (2,014) — Xtreme Recon Pkg

Overall Height	73.6 (1,868.4)

74.5 — Rubicon 392

75.5 — Xtreme Recon Pkg

### CLEARANCES

| Clearances | 245/75R17 Standard on Sport | 255/70R18 Standard on Sahara | LT285/70R17 Standard on Rubicon | LT315/70R17 Optional on Rubicon 4-door |
| --- | --- | --- | --- | --- |
| Approach Angle (degrees) | 41.4 | 41.8 | 44 (2-door) 43.9 (4-door) 44.5 (Rubicon 392) | 47.4 46.7 (Rubicon 392) |
| Breakover Angle (degrees) | 25 (2-door) 20.3 (4-door) | 21 | 27.8 (2-door) 22.6 (4-door, Rubicon 392) | 26.7 24.5 (Rubicon 392) |
| Departure Angle (degrees) | 35.9 (2-door) 36.1 (4-door) | 36.1 | 37 37.5 (Rubicon 392) | 40.4 39.8 (Rubicon 392) |
| Ground Clearance (inches) | 9.7 | 10 | 10.8 10.3 (Rubicon 392) | 12.9 11.1 (Rubicon 392) |
| Ramp Travel Index (Front Sta-bar disconnected) | N/A | N/A | 890 (2-door) 770 (4-door) 730 (Rubicon 392) | 712 (Rubicon 392) |

*Measurements the same for both 2- and 4-door models unless specified

### CURB WEIGHT

| CURB WEIGHT | 2.0-liter (2- and 4-door) | 3.0-liter (4-door) | 3.6-liter (2- and 4-door) | 6.4-liter (4-door) |
| --- | --- | --- | --- | --- |
| Wrangler Sport |  |  |  |  |
| Automatic Transmission (lbs./kg) | 3,948 (1,791) / 4,196 (1,903) | 4,654 (2,111) | 3,919 (1,778) / 4,167 (1,890) | N/A |
| Manual Transmission (lbs./kg) | N/A | N/A | 3,948 (1,791) / 4,196 (1,903) | N/A |
| Wrangler Sahara |  |  |  |  |
| Automatic Transmission (lbs./kg) | 4,406 (1,999) | 4,737 (2,149) | 4,406 (1,999) | N/A |
| Manual Transmission (lbs./kg) | N/A | N/A | 4,263 (1,934) | N/A |
| Wrangler Rubicon |  |  |  |  |
| Automatic Transmission (lbs./kg) | 4,222 (1,915) / 4,478 (2,031) | 4,862 (2,205) | 4,193 (1,902) / 4,449 (2,018) | 5,098 (2.312) |
| Manual Transmission (lbs./kg) | N/A | N/A | 4,222 (1,915) / 4,478 (2,031) | N/A |

*Weights differ by transmission called out

### Accommodations

Seating Capacity (front/rear) 	2/2 (2-door), 2/3 (4-door)

SAE Total Interior Passenger Volume,
(cu. ft. / cu. m)	103.7 (2.94)

Front

Headroom	40.8 (1,036) hardtop, 42.6 (1,083) soft top

Legroom	41.2 (1,046)

Shoulder room	55.8 (1,417)

Hip room	53.9 (1,370)

Seat travel	8.1 (206)

EPA front volume index	53.8 (1.52)

Rear

Headroom	40.3 (1,023) hardtop, 41.7 (1,059) soft top

Legroom	35.7 (904.2) — 2-door, 38.3 (974) — 4-door,

Knee clearance	61.6 (1,565.9)

Shoulder room	57.7 (1,458) — 2-door, 55.8 (1,417) — 4-door

Hip room	45 (1,143) — 2-door, 56.7 (1,440) — 4-door,

EPA rear seat volume index 
(cu. ft. / cu. m) 	49.9 (1.40)

Liftover height	29.7 (756.4), 30.8 (782) — Rubicon 392

Load floor height	30 (762.1)

Maximum cargo width at liftgate opening	47.9 (1,218.7)

Minimum cargo width at liftgate opening	41.1 (1,052)

Maximum cargo height at liftgate opening	36.0 (942.3)

Minimum cargo height at liftgate opening	35.7 (907.8)

Distance between wheelhouse interior trim	45.0 (1,143)

SAE Cargo Volume

Rear seat folded	72.4 cu. ft. (2.05)

Rear seat upright	31.7 cu. ft. (0.89)

Total passenger plus cargo volume, 
(cu. ft. / cu. m)	150.4 / 4.26

### WHEELS

Availability	Standard (Sport)

Type and material	Painted steel, Low Gloss Black painted

Size	17 x 7.5 in.

Availability	Standard (Sport S)

Type and material	Fully painted cast-aluminum with High Gloss Tech Silver

Size	17 x 7.5 in.

Availability	Optional (Sport S)

Type and material	Painted and polished cast-aluminum with Low Gloss Granite Crystal

Size	17 x 7.5 in.

Availability	Standard (Sahara)

Type and material	Painted spokes with polished cast-aluminum

Size	18 x 7.5 in.

Availability	Optional (Sahara)

Type and material	Painted High Gloss Technical Gray with polished cast aluminum

Size	18 x 7.5 in.

Availability	Standard (Rubicon)

Type and material	Painted pocket with Mid-Gloss paint and polished cast aluminum

Size	17 x 7.5 in.

Availability	Optional (Rubicon)

Type and material	Painted pocket with Mid-Gloss paint and polished cast aluminum

Size	17 x 7.5 in.

Availability	Standard (Rubicon 392)

Type and material	Beadlock capable, Low Gloss Smoked Bronze paint and Low Gloss Black trim ring with Low Gloss Black bolts cast aluminum

Size	17 x 7.5 in.

Availability	Optional (Rubicon/Sport 4-door)

Type and material	Beadlock capable, Low Gloss Granite Crystal Paint and Low Gloss Black trim ring with Low Gloss Black bolts cast aluminum

Size	17 x 8 in.

Availability	Optional (Rubicon 392)

Type and material		Beadlock capable, Low Gloss Smoked Bronze paint and Low Gloss Black trim ring with Low Gloss Black bolts cast aluminum

Size	17 x 8 in.

### TIRES

Availability	Standard (Sport)

Size and type 	P245/75R17, on/off-road, black sidewall

Mfr. and model	Michelin LTX MS2, Bridgestone Dueler HT 685, Bridgestone Dueler AT RH-S, Goodyear Wrangler Adventure AT

Revs per mile (km)	665 (413) — Michelin LTX MS2

661 (411) — Bridgestone Dueler HT 685 & AT RH-S

668 (415) — Goodyear Wrangler Adventure AT

Availability	Standard (Sahara)

Size and type	P255/70R18, on/off-road, OWL

Mfr. and model	Bridgestone Dueler HT 685, Bridgestone Dueler AT RH-S, 
Goodyear Wrangler Adventure AT

Revs per mile (km)	649 (403)

Availability	Standard (Rubicon, Rubicon 392)

Size and type	LT285/70R17C, on/off-road, black sidewall

Mfr. and model	BF Goodrich KO2 All-terrain

Revs per mile (km) 	645 (401)

Availability	Optional (Rubicon)

Size and type	LT285/70R17C

Mfr. and model 	Falken Wildpeak M/T

Revs per mile (km) 	617 (383)

Availability		Optional (Rubicon and Rubicon 392)

Size and type	LT285/70R17C

Mfr. and model 	Falken Wildpeak M/T

Revs per mile (km) 	617 (383)

Availability 	Optional (Rubicon 4-door and Rubicon 392)

Size and type	LT315/70R17C, on/off-road, black sidewall

Mfr. and model 	BF Goodrich KO2 All-terrain

Revs per mile (km) 	604 (376)

Four-wheel Anti-lock Brakes	Standard

Electronic Stability Control	Standard

All-speed Traction Control	Standard

### TOWING CAPACITIES

#### WRANGLER SPORT (4-door)

| Engine | Trans Type | Transmission | Axle Ratio | Payload | Base Wt. | Base Wt. Front | Base Wt. Rear | GAWR Front | GAWR Rear | GCWR | Max Trailer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 3.45 | 1233 | 4167 | 2217 | 1950 | 2700 | 3000 | 8117 | 3500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 3.45 | 1204 | 4196 | 2246 | 1950 | 2700 | 3000 | 8117 | 3500 |
| 3.0-liter V-6 diesel | A8 | 8HP75 8-speed automatic | 3.73 | 1136 | 4654 | 2494 | 2160 | 3100 | 3100 | 8427 | 3500 |
| 2.0-liter I-4 gas | A8 | 850RE 8-speed automatic | 3.45 | 1204 | 4196 | 2298 | 2038 | 2700 | 3000 | 8117 | 3500 |

GCWR = Base Weight + Maximum Trailer Weight + 150 lbs. for driver.

#### WRANGLER SAHARA (4-door)

| Engine | Trans Type | Transmission | Axle Ratio | Payload | Base Wt. | Base Wt. Front | Base Wt. Rear | GAWR Front | GAWR Rear | GCWR | Max Trailer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 3.45 | 1237 | 4263 | 2255 | 2007 | 2700 | 3000 | 8117 | 3500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 3.45 | 1208 | 4406 | 2321 | 2085 | 2700 | 3000 | 8117 | 3500 |
| 3.0-liter V-6 diesel | A8 | 8HP75 8-speed automatic | 3.73 | 1188 | 4737 | 2578 | 2159 | 3100 | 3100 | 8427 | 3500 |
| 2.0-liter I-4 gas | A8 | 850RE 8-speed automatic | 3.45 | 1208 | 4406 | 2321 | 2085 | 2700 | 3000 | 8117 | 3500 |

GCWR = Base Weight + Maximum Trailer Weight + 150 lbs. for driver.

#### WRANGLER RUBICON (4-door)

| Engine | Trans Type | Transmission | Axle Ratio | Payload | Base Wt. | Base Wt. Front | Base Wt. Rear | GAWR Front | GAWR Rear | GCWR | Max Trailer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 4.10 | 1351 | 4449 | 2339 | 2110 | 3100 | 3100 | 8117 | 3500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 4.10 | 1322 | 4478 | 2368 | 2110 | 3100 | 3100 | 8117 | 3500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 4.56 | 1144 | 4656 | 2425 | 2231 | 3100 | 3550 | 8117 | 3500 |
| 3.6-liter V-6 gas | M6 | D478 6-speed manual | 4.88 | 1351 | 4449 | 2339 | 2110 | 3100 | 3100 | 8117 | 3500 |
| 3.6-liter V-6 gas | A8 | 850RE 8-speed automatic | 4.88 | 1322 | 4478 | 2368 | 2110 | 3100 | 3100 | 8117 | 3500 |
| 3.0-liter V-6 diesel | A8 | 8HP75 8-speed automatic | 3.73 | 1233 | 4682 | 2636 | 2226 | 3100 | 3100 | 8427 | 3500 |
| 2.0-liter I-4 gas | A8 | 850RE 8-speed automatic | 4.10 | 1322 | 4478 | 2368 | 2110 | 3100 | 3100 | 8117 | 3500 |
| 6.4-liter V-8 gas | A8 | 8HP75 8-speed automatic | 3.73 | 1002 | 5098 | 2756 | 2342 | 3450 | 3550 | 6100 | 3500 |
| 6.4-liter V-8 gas | A8 | 8HP75 8-speed automatic | 4.56 | 1028 | 5222 | 2773 | 2449 | 3450 | 3550 | 6250 | 3500 |

GCWR = Base Weight + Maximum Trailer Weight + 150 lbs. for driver.

###