# 2023 Ram Commercial ProMaster

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Body Styles and Wheelbase	ProMaster 1500 Cargo Van 118-in. wheelbase low roof

ProMaster 1500 Cargo Van 136-in. wheelbase low roof

ProMaster 1500 Cargo Van 136-in. wheelbase high roof

ProMaster 2500 Cargo Van 136-in. wheelbase low roof

ProMaster 2500 Cargo Van 136-in. wheelbase high roof

ProMaster 2500 Cargo Van 159-in. wheelbase high roof

ProMaster 2500 Window Van 159-in. wheelbase high roof

ProMaster 3500 Cargo Van 136-in. wheelbase low roof

ProMaster 3500 Cargo Van 136-in. wheelbase high roof

ProMaster 3500 Cargo Van 159-in. wheelbase high roof

ProMaster 3500 Cargo Van 159-in. wheelbase high roof extended body

ProMaster 3500 Window Van 159-in. wheelbase high roof extended body

ProMaster 3500 Chassis Cab 136-in. wheelbase

ProMaster 3500 Chassis Cab 159-in. wheelbase

ProMaster 3500 Chassis Cab 159-in. wheelbase extended body

ProMaster 3500 Chassis Cab Cutaway 136-in. wheelbase

ProMaster 3500 Chassis Cab Cutaway 159-in. wheelbase

ProMaster 3500 Chassis Cab Cutaway 159-in. wheelbase extended frame

Assembly Plant	Saltillo Van Assembly, Coahuila, Mexico

### BODY AND CHASSIS

Layout	Transverse, front engine, front-wheel drive (FWD)

Construction	Unibody and ladder frame

### ENGINE: 3.6-LITER PENTASTAR V-6

Type and Description	60-degree gasoline V-6

Displacement	220 cu. in. (3,604 cu. cm)

Bore x Stroke	3.78 x 3.27 (96.0 x 83.0)

### ENGINE: 3.6-LITER PENTASTAR V-6 (CONTINUED)

Valve System	Chain-driven DOHC, 24 valves and hydraulic end-pivot roller rockers

Fuel Injection	Sequential, multiport, electronic, returnless

Construction	Aluminum deep-skirt block, aluminum alloy heads

Compression Ratio	11.3:1

Power 	276 hp (205 kW) at 6,400 rpm

Torque 	250 lb.-ft. (339 N•m) at 4,400 rpm

Max. Engine Speed	6,400 rpm (electronically limited)

Fuel Requirement	Unleaded regular, 87 octane

Oil Capacity	6.0 quarts (5.7 liters)

Coolant Capacity	14.0 quarts (13.25 liters)

Emission Controls	Dual three-way catalytic converters, heated oxygen sensors

### TRANSMISSION: 948TE TORQUEFLITE NINE-SPEED AUTOMATIC, FRONT-WHEEL DRIVE

Availability	Standard with 3.6-liter Pentastar V-6

Description	Nine-speed FWD, electronically controlled automatic overdrive transmission with torque converter clutch. Clutch-to-clutch architecture, with integral electro/hydraulic control module

Gear Ratios

1st	4.71

2nd	2.84

3rd	1.91

4th	1.38

5th	1.00

6th	0.81

7th	0.70

8th	0.58

9th	0.48

Reverse	3.83

Spread	9.81

Final Drive Ratio	4.08

### AXLES

Front	Independent front suspension

Rear – Cargo Van	Solid axle

Rear – Chassis Cab and Cut Away	Solid axle

### ELECTRICAL SYSTEM

Alternator	Cargo Van: 180-amp standard; 220-amp optional

Chassis Cab and cut-away: 220-amp standard

Battery	95-amp with 800 CCA – Standard

100-amp with 850 CCA – Optional

### SUSPENSION

Front	MacPherson struts with stabilizer bar

Rear	Solid axle with leaf springs Hotchkiss suspension

### BRAKES

Front Rotor (diameter x width)	11.8 x 1.26 (300 x 32)

Rear Rotor (diameter x width)	11.8 x 0.87 (300 x 22)

Front Caliper, Lining	Cast iron, dual piston (50 mm and 52 mm)

Rear Caliper, Lining	Cast iron, single piston (51 mm)

### STEERING

Type	Rack and pinion with electric power steering

Turns lock to lock 	3.9

Steering Ratio 	18.85

Turning diameter	36.3 ft

###