# 2023 Wagoneer L / Grand Wagoneer L

## SPECIFICATIONS

Specifications are based on the latest product information available at the time of publication.
All dimensions are in inches (millimeters) unless otherwise noted.
All dimensions measured at curb weight with standard tires and wheels.

### GENERAL INFORMATION

Vehicle Type	Four-door sport-utility vehicle

Assembly Plant	Warren Truck Assembly Plant, Warren, Michigan

EPA Vehicle Class	Multipurpose vehicle

Introduction Date	Second half 2022 as a 2023 model

### BODY AND CHASSIS

Layout	Front engine, rear- or four-wheel drive

Construction	Steel frame

### ENGINE: 3.0-liter HURRICANE TWIN TURBO

Availability	Standard — Wagoneer L

Type and Description	In-line six-cylinder (I-6), Twin Turbocharged, direct injection, gasoline

Displacement, cu. in. (cu.cm.)	183 cu. in. (2.993 cu. cm)

Bore x Stroke, in. (mm)	3.31 x 3.54 (84 x 90)

Valve System	DOHC, dual independent valve timing, hydroformed tubular camshafts, 24 valves with sodium-filled exhaust valves, roller finger followers and hydraulic lash adjusters

Fuel Injection	Gasoline direct injection, centrally located injectors, single high pressure pump providing up to 5,075 psi (350 bar) to a single fuel rail

Construction	Deep-skirt cast-aluminum block with cross-bolted steel main bearing caps, low friction plasma spray-bore coated cylinders, structural die-cast aluminum alloy oil pan, aluminum alloy head with pent-roof combustion chambers, forged and twisted steel crankshaft, forged steel connecting rods and cast aluminum alloy pistons with DLC coated piston pins

Compression Ratio	10.4:1

Power (SAE J2723)	420 hp (313 kW) @ 5,200 rpm

Torque (SAE J2723)	468 lb.ft. (635 Nm) @ 3,500 rpm

Max. Engine Speed	5,800 rpm (electronically limited)

Fuel Requirement	Unleaded mid-grade, 89 octane (R+M)/2 — recommended

Unleaded regular, 87 octane (R+M)/2 — acceptable

### ENGINE: 3.0-liter HURRICANE TWIN TURBO (CONTINUED)

Oil Capacity	7.5 quarts (7.1 liters) oil change with filter

Coolant Capacity qt. (liter)	High Temperature Circuit: 16.6 (15.7) 
Low Temperature Circuit: 2.85 (2.70)

Factory Oil Fill 	Pennzoil Ultra Platinum 0W-20 GF6+ (API SP)

Emission Controls	Dual close-coupled two-stage three-way catalytic converters, quad heated oxygen sensors, cooled exhaust gas recirculation and internal engine features

EPA Fuel Economy mpg estimates 
(city/hwy/combined)	4x2: 17/24/20, 4x4: 16/23/19

Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

### ENGINE: 3.0-liter HURRICANE TWIN TURBO 510

Availability	Standard — Grand Wagoneer L

Type and Description	In-line, liquid-cooled

Displacement, cu. in. (cu. cm.)	183 (2,993)

Bore x Stroke	3.31 x 3.54 (84 x 90)

Valve System	DOHC, dual independent valve timing, hydroformed tubular camshafts, 24 valves with sodium-filled exhaust valves, roller finger followers and hydraulic lash adjusters

Fuel Injection	Gasoline direct injection, centrally located injectors, dual high-pressure pumps providing up to 5,075 psi (350 bar) to a single fuel rail

Construction	Deep-skirt cast-aluminum block with cross-bolted steel main bearing caps, low friction plasma spray-bore coated cylinders, structural die-cast aluminum alloy oil pan, aluminum alloy head with pent-roof combustion chambers, forged and twisted steel crankshaft, forged steel connecting rods and forged aluminum alloy pistons with DLC coated piston pins

Compression Ratio	9.5:1

Power (SAE J2723)	510 hp (375 kW) @ 5,700 rpm

Torque (SAE J2723)	500 lb.-ft. (678 N•m) @ 3,500 rpm

Max. Engine Speed	6,100 rpm (electronically limited)

Fuel Requirement	Premium 91 octane (R+M)/2 — required

Oil Capacity, qt. (liter) 	7.5 (7.1) oil change with filter

Coolant Capacity, qt. (liter)	High Temp. Circuit:  16.6 (15.7) Low Temp. Circuit:  2.85 (2.70)

Factory Oil Fill 	Pennzoil Ultra Platinum 0W-40 API SP MS-A0921

Emission Controls 	Dual close-coupled two-stage three-way catalytic converters, quad heated oxygen sensors and internal engine features(a)

Exhaust Type 	Dual exhaust with black dual hidden tail pipes

### ENGINE: 3.0-liter HURRICANE TWIN TURBO 510 (continued)

EPA Fuel Economy mpg estimates 
(city/hwy/combined)	14/19/16

Assembly Plant	Saltillo Engine Plant, Saltillo, Mexico

(a) Meets Federal Tier 3, Bin 70 emissions requirements; marketed in California as a LEV III ULEV70 (Ultra Low Emission Vehicle).

### TRANSMISSION: TORQUEFLITE 8HP75 AUTOMATIC, EIGHT-SPEED OVERDRIVE

Availability	Standard — 3.0-liter Hurricane Twin Turbo, Hurricane Twin Turbo 510

Description	Five clutches and four planetary gear set architecture. 7.03 overall ratio spread. Start/stop and electronic range select capability. Electronically modulated torque converter clutch with torsional turbine damper

Gear Ratios

1st	4.71

2nd	3.14

3rd	2.11

4th	1.67

5th	1.29

6th	1.00

7th	0.84

8th	0.67

Reverse 	3.30

### TRANSFER CASE: 1-SPEED

Availability 	Optional — 3.0-liter Hurricane Twin Turbo

Type 	Single-speed

Operating Mode 	Full-time active 4x4

Torque Split, Front/Rear	Variable — Auto, Tow, Snow, Mud, Sand, Rock, Sport

Low Range Ratio 	None

### TRANSFER CASE: 2-SPEED

Availability 	Optional — 3.0-liter Hurricane Twin Turbo

Standard — 3.0-liter Hurricane Twin Turbo 510

Type 	Two-speed electronically shifted

Operating Modes 	4x4 Low Active, Neutral, full-time active 4x4

Low Range Ratio	2.64

Torque Split, Front/Rear 	Variable — Auto, Tow, Snow, Mud, Sand, Rock, Sport

### AXLES

Front

Differential Type	Open

Availability 	Standard on 4x4 models

Ring Gear Diameter	8.5 (215)

Axle Ratios	3.55:1 — Standard with 3.0-liter Hurricane Twin Turbo

3.92:1 — Optional with 3.0-liter Hurricane Twin Turbo, standard with 3.0-liter Hurricane Twin Turbo 510

Rear

Differential Type 	Limited Slip or Electronic Limited Slip (ELSD)

Availability 	Limited Slip Differential — Standard with 3.0-liter Hurricane Twin Turbo

Electronic Limited Slip Differential (ELSD) — Optional with 3.0-liter Hurricane Twin Turbo, standard with 3.0-liter Hurricane Twin Turbo 510

Ring Gear Diameter (in/mm) 	10.2 (260mm)

Axle Ratios 	3.55:1 — Standard with 3.0-liter Hurricane Twin Turbo

3.92:1 — Optional with 3.0-liter Hurricane Twin Turbo, standard with 3.0-liter Hurricane Twin Turbo 510

### ELECTRICAL SYSTEM

Alternator	240-amp — Standard with 3.0-liter, Hurricane Twin Turbo and Hurricane Twin Turbo 510

Battery	Maintenance-free

H7 AGM 700 CCA — Standard with 3.0-liter, Hurricane Twin Turbo and Hurricane Twin Turbo 510 with Aux AGM 200 CCA

### SUSPENSION

Front	Short- and long-arm independent with hybrid steel composite upper control arm, aluminum lower control arm, aluminum knuckle, coil springs with monotube shocks or Quadra-lift air suspension with 
semi-active damping, solid or hollow stabilizer bar

Rear	Five-link independent rear suspension, coil springs with monotube load leveling shocks or Quadra-lift air suspension with semi-active damping, solid or hollow stabilizer bar, cast aluminum links (tension, compression, camber, toe), and high strength steel spring link

### STEERING

Type	Electric rack-and-pinion steering

Overall Ratio	16.0:1

Turning Diameter (curb-to-curb) (ft./m)	42.2 / 12.9

Steering Turns (lock-to-lock)	2.97

### BRAKES

Type	Electronic power brakes (eBooster), anti-lock brake system (ABS)

Availability	Standard

Front

Rotor size and type	14.88 x 1.18 (378 x 30) vented disc

Caliper size and type	2.24 (57) two-piston pin-slider caliper

Swept area, (sq. in. / sq. cm)	100.9 (651.5)

Rear

Rotor size and type 	14.76 x 0.87 (375 x 22) solid disc

Caliper size and type	2.24 (57) single-piston pin-slider caliper with EPB

Swept area, (sq. in. / sq. cm)	79.2 (511.6)

### DIMENSIONS AND CAPACITIES

Wheelbase	130.0 (3,303)

Track, Front	68.4 (1,738)

Track, Rear	68.3 (1,734)

Overall Length	226.7 (5,759)

Overall Width (width at mirrors)	94.0 (2,388)

Body Width	83.6 (2,124)

Overall Height (at roof rail / at antenna)	79.6 (2023) / 77.3 (1963)

Load Floor Height	34.0 (864) — standard suspension

31.9 (811) — air suspension

Sill Step Height	22.8 (578) — standard suspension

21.1 (535) — air suspension

Ground Clearance (with 275/55R20 tire)	8.3 (210) — standard suspension

10.0 (253) — air suspension (Pos#2)

Chassis (fuel tank) 	9.6 (245) — standard suspension

10.4 (265) — air suspension (Pos#2)

Front Axle 	9.5 (242) — standard suspension

11.5 (293) — air suspension (Pos#2)

Rear Axle 	8.9 (227) — standard suspension

10.9 (278) — air suspension (Pos#2)

### DIMENSIONS AND CAPACITIES (continued)

Approach Angle (degrees)	21.5 — standard suspension

25.0 — air suspension (Pos#2)

Ramp Breakover Angle (degrees)	17.8 — standard suspension

20.4 — air suspension (Pos#2)

Departure Angle (degrees)	18.8 — standard suspension

21.3 — air suspension (Pos#2)

Frontal Area sq. ft. (sq. m)	37.2 (3.46)

Drag Coefficient (Cd)	0.352 — Wagoneer L

0.364 — Grand Wagoneer L

Aero CdA sq. ft. (sq. m)	13.1 (1.22) — Wagoneer L

13.5 (1.26) — Grand Wagoneer L

Fuel Tank Capacity, (gallons / liter)	30.5 (115.4)

### ACCOMMODATIONS

Seating Capacity	Wagoneer L

Standard — 2/3/3 (including 2nd row bench)

Optional — 2/2/3 (including captain’s chairs)

Grand Wagoneer L

Standard — 2/2/3 (including captain’s chairs)

Optional — 2/3/3 (including 2nd row bench)

SAE Total Interior Passenger Volume,
(cu. ft. / cu. m)	179.3 (5.1) Wagoneer L / 172.8 (4.9) Grand Wagoneer L

Front

Headroom	41.3 (1049) Wagoneer L / 39.3 (998) Grand Wagoneer L

Legroom	40.9 (1040)

Shoulder room	66.1 (1680)

Hip room	63.4 (1610)

Seat travel	8.1 (207)

EPA front volume index (cu. ft. / cu. m)	64.7 (1.8) Wagoneer L / 61.6 (1.7) Grand Wagoneer L

Cargo volume behind first-row seats
(cu. ft. / cu. m)	130.9 (3.7) Wagoneer L / 112.9 (3.2) Grand Wagoneer L

2nd Row

Headroom	40.0 (1016) Wagoneer L / 38.4 (975) Grand Wagoneer L

Legroom	42.7 (1085)

Knee clearance	6.7 (170) Wagoneer L / 5.6 (143) Grand Wagoneer L

### ACCOMMODATIONS (CONTINUED)

Shoulder room	65.2 (1657)

Hip room	63.0 (1599)

EPA 2nd row seat volume index
(cu. ft. / cu. m)	64.5 (1.8) Wagoneer L / 61.9 (1.8) Grand Wagoneer L

Cargo volume behind second-row seats
(cu. ft. / cu. m)	85.3 (2.4) Wagoneer L / 88.8 (2.5) Grand Wagoneer L

3rd Row

Headroom	39.1 (992) Wagoneer L / 38.5 (977) Grand Wagoneer L

Legroom	36.6 (930)

Knee clearance	1.9 / 47

Shoulder room	64.4 (1635)

Hip room	51.6 (1310)

EPA 3rd row seat volume index
(cu. ft. / cu. m)	50.1 (1.4) Wagoneer L / 49.3 (1.4) Grand Wagoneer L

Cargo volume behind third-row seats
(cu. ft. / cu. m)	42.1 (1.2) Wagoneer L / 44.2 (1.3) Grand Wagoneer L

Total passenger plus cargo volume
(cu. ft. / cu. m)	221.4 (6.3) Wagoneer L / 217.0 (6.1) Grand Wagoneer L

### WEIGHTS (Estimates)

| WHEEL BASE | DRIVE SYSTEM | MODEL | ENGINE | GVWR lbs. (kg) | CURB WEIGHT(B) | PAYLOAD lbs. (kg)(C) | WEIGHT DISTRIBUTION, F/R |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 130 in. | 2WD | Wagoneer L | 3.0L | 7400 | 6069 | 1410 | 51/49 |
|  |  | Wagoneer L Series II | 3.0L | 7400 | 6068 | 1410 | 51/49 |
|  |  | Wagoneer L Series III | 3.0L | 7500 | 6107 | 1470 | 51/49 |
|  | 4WD | Wagoneer L | 3.0L | 7700 | 6297 | 1500 | 52/48 |
|  |  | Wagoneer L Series II | 3.0L | 7600 | 6326 | 1360 | 52/48 |
|  |  | Wagoneer L Series III | 3.0L | 7800 | 6335 | 1550 | 52/48 |
|  |  | Grand Wagoneer L | 3.0L 510 | 7950 | 6621 | 1420 | 51/49 |
|  |  | Grand Wagoneer L Series II | 3.0L 510 | 7950 | 6685 | 1350 | 51/49 |
|  |  | Grand Wagoneer L Series III | 3.0L 510 | 7950 | 6704 | 1330 | 51/49 |

(b) Curb weight includes standard equipment and full quantities of fuel, lubricant and coolant.
(c) Payload is the maximum allowable weight of driver, passengers, cargo, and options, rounded to the nearest 10 lbs. (5kg).

### Wheels

Availability	Standard on Wagoneer L

Type and Material	Fully Painted Satin Carbon Finish, six spoke

Size	18 x 8

Availability 	Optional on Wagoneer L

Type and Material	Machined cast aluminum with Black Noise painted pockets, six spoke

Size	20 x 9

Availability 		Standard on Wagoneer L Series II

Type and material	Machined cast aluminum with Black Noise painted pockets, six spoke

Size	20 x 9

Availability	Standard on Wagoneer L Series III

Type and material	Machined cast aluminum with Satin granite crystal pockets, six spoke

Size	20 x 9

Availability	Standard on Grand Wagoneer L

Type and material	Machined cast aluminum with Baltic Gray painted pockets

Size	20 x 9

Availability 	Standard on Grand Wagoneer L Series II, optional on 
Grand Wagoneer L

Type and material	Polished aluminum with Luster Gray painted pockets

Size	22 x 9

Availability	Standard on Grand Wagoneer L Series III

Type and material	Polished aluminum with Lights Out painted pockets

Size	22 x 9

Availability	Optional on Wagoneer L Series II, optional on Wagoneer L Series III

Type and material	Polished aluminum with Black Noise painted pockets

Size	22 x 9

Availability	Optional on Grand Wagoneer L Series II

Type and material	Painted cast aluminum Satin Carbon with chrome inserts

Size	22 x 9

Availability		Optional on Grand Wagoneer L Series III

Type and material	Machined cast aluminum with Black Noise painted pockets and brushed nano steel inserts

Size	22 x 9

Availability	Optional with Off-road Package

Type and material	Machined cast aluminum with Black Noise painted pockets

Size	18 x 8

### Wheels (CONTINUED)

Availability	Standard Carbide

Type and material	Cast aluminum fully painted with Gloss black

Size (in.)	20 x 9

Availability	Optional Carbide

Type and material	Cast aluminum fully painted with Gloss Black

Size (in.) 	22 x 9

Availability	Standard Obsidian

Type and material	Fully painted with Gloss Black smooth inserts

Size (in.)	22 x 9

### TIRES

Availability	Standard — Wagoneer L

Size and type	275/65R18

Mfr. And model	Bridgestone Dueler H/T All Season

Revs per mile	653

Availability	Standard — Wagoneer L Series II, Wagoneer L Series III

Size and type	275/55R20

Mfr. And model	Nexen Roadian HTX RH5 All Season

Revs per mile	656

Availability 	Standard – Grand Wagoneer L; Packaged – HD Trailer Tow, 
Premium Group I

Size and type	275/55R20

Mfr. And model	Bridgestone Dueler H/L Alenza All Season

Revs per mile	655

Availability	Standard — Grand Wagoneer L Series II, Grand Wagoneer L Series III; 
Packaged — Premium Group I

Size and type	285/45R22XL

Mfr. and model	Goodyear Eagle Touring All Season/Performance

Revs per mile	639

Availability 	Packaged — Advanced All Terrain Group

Size and type 	275/65R18

Mfr. and model 	Firestone Destination AT2 All Terrain

Revs per mile 	651

### TIRES (CONTINUED)

Parking Brake Type	Electric power steering (standard)

Four-wheel Anti-lock Brakes	Standard

Electronic Stability Control	Standard

All-speed Traction Control	Standard

Brake Assist	Standard

### TRAILER TOWING

| WHEELBASE | DRIVE SYTEM | ENGINE | AXLE RATIO | MAXIMUM TRAILER WEIGHT lbs. |
| --- | --- | --- | --- | --- |
| 130 in. | 2WD | 3.0L | 3.92 | 10,000 |
|  | 4WD | 3.0L | 3.92 | 9,850 |
|  |  | 3.0L 510 | 3.92 | 9,450 |

###